### Supplementary

S1. Evaluation datasets

## A: Spin-up and pre-industrial simulations

The historic simulation is preceded by three steps of spin-up to equilibrate plant and soil organic C, N and P to pre-industrial conditions. The spin-up steps are forced by looped climate fields over the period 1901-1920, fixed pre-industrial atmospheric CO2 (286 ppm) and land cover maps. We run the spin-up 1 over 200 years with forced C:N:P ratios of plant and soil, followed by two iterations of an analytic spin-up (40 years for each iteration) (Vuichard et al., 2019). This facilitates the accumulation of soil organic C, N and P and approaches the equilibrium for plant productivity. We then run the spin-up 2 from the last year of the spin-up 1 with dynamic C:N:P ratios of plant and soils for 10k years, to reach a quasi-equilibrium for plant and soil N and P pools. In the last step, crop management (harvest and fertilizer) was activated for 200 years.

## B: Global Observation-based Land-ecosystems Utilization Model of Carbon, Nitrogen and Phosphorus (GOLUM-CNP)

Global Observation-based Land-ecosystems Utilization Model of Carbon, Nitrogen and Phosphorus (GOLUM-CNP) v1.0 is a data-driven modeling of steady-state carbon, nitrogen and phosphorus cycles for present day (2001-2016) conditions. It combines the CARbon DAta MOdel fraMework (CARDAMOM) data-constrained C-cycle analysis with spatially explicit data-driven estimates of N and P inputs and losses and with observed stoichiometric ratios. We extracted the following variables from GOLUM: global GPP and NPP, N and P in- and out- fluxes (e.g. BNF, N and P deposition, N and P leaching) on global and biome scale, biome-specific N and P resource use efficiencies, N and P openness, and N and P turnover rate. GOLUM-CNP only computed fluxes for non-cropland biomes. To compare the global total stocks and flux budgets of C, N and P from GOLUM-CNP with those from ORCHIDEE-CNP, we multiply the stock and flux in each grid cell of GOLUM-CNP by the non-cropland fraction from the MODIS land cover map (version MCD12C1v006, https://lpdaac.usgs.gov/products/mcd12c1v006/) and sum over all the grid cells.

## C: Evaluation datasets for the C budget

### C1 GPP and NPP from MODIS

MODIS GPP is derived from climate and satellite data. It was estimated at 1 km spatial resolution from a light-use efficiency (LUE) model, as part of the operational MODIS algorithms (Running et al., 2004) using meteorological data from NASA Data Assimilation Office and detailed vegetation information (land cover and FPAR) derived from the Moderate Resolution Imaging Spectroradiometer (MODIS) satellite from 2000 to present (Running et al., 2004; Zhao et al., 2005; Turner et al., 2006).

### C2 MTE GPP

MTE GPP is a data-oriented product derived by extrapolating the GPP estimates from a network of flux-tower of 178 sites in space and time using climate data and remotely sensed fAPAR data from 1982 to 2008 (Jung et al., 2009). This latter monthly GPP product covers the period 1982-2011 at 0.5° spatial resolution, but many of the underlying flux tower observations cover much shorter periods.

### C3 BESS-GPP

Breathing Earth System Simulator (BESS) is a simplified process-based model that couples atmosphere and canopy radiative transfers, canopy photosynthesis, transpiration, and energy balance. This process-based model uses the MODIS Atmosphere products to calculate atmospheric radiative transfer. This product provides gridded GPP on a monthly scale from 2000-2015 at 0.5° spatial resolution.

### C4 BETHY-NPP

Biosphere Energy-Transfer Hydrology (BETHY)/DLR is a Soil-Vegetation-Atmosphere (SVAT) model to simulate photosynthesis of terrestrial ecosystems, which is operated at the German Aerospace Center (DLR). It was adapted from the BETHY scheme to be driven by remote sensing data (leaf area index (LAI) and land cover information) and meteorology (Wißkirchen et al., 2013). This product provides gridded global NPP on a monthly scale from 2000-2009 at 0.008° spatial resolution.

### C5 GIMMS-NPP

GIMMS-NPP is a NPP product derived from the MODIS NPP algorithm driven by long-term Global Inventory Modeling and Mapping Studies (GIMMS) FPAR and LAI data (Smith et al., 2016). The uncertainty bounds were given by using a wide range of parameter combinations. This global satellite-derived NPP is available from 1982-2011 with a spatial resolution of 0.5° x 0.5°.

### C6 Net biome productivity from GCP and inversion data

Estimates are based on 16 DGVMs involved in GCP Trendy v6 were forced by CRU-NCEP temperature, precipitation, and cloud cover fields (transformed into incoming surface radiation) based on observations and provided on a 0.5° × 0.5° grid and updated to 2016. We compared ORCHIDEE-CNP with Trendy v6 simulations instead of v7 because ORCHIDEE-CNP was part of the v7 ensemble.

We used three sets of CO2 inversion data from CarbonTracker (van der Laan-Luijkx et al., 2017), Jena CarboScope (Rödenbeck, 2005), and CAMS (Chevallier et al., 2005). They all used atmospheric inversion methods (based on the same Bayesian inversion principles) and atmospheric CO2 observations from various flask and in situ networks to constrain the location of the combined total surface CO2 fluxes from all sources. The three systems used different transport models, which was demonstrated to be a driving factor behind differences in atmospheric-based flux estimates, and specifically for the global distribution (Stephens et al., 2007).

## D: Evaluation datasets for the N and P budget

### D1 Biological Nitrogen fixation (BNF) from Peng et al. (2019)

We used the global gridded BNF (both symbiotic and asymbiotic) from Peng et al., (2019) for 2001-2010. They calculated BNF as a function of soil N, LAI, labile inorganic soil P, temperature, etc (Wang et al., 2007; Houlton et al., 2008) which were constrained by the observation-based estimates of C:N ratios of the various plant pools, soil microbial biomass and organic matter under present conditions.

### D2 N and P loads to rivers on basin scale by GlobalNEWS2 model

Global Nutrient Export from Water-Sheds (GLOBNEWS) model generates global spatially explicit of N and P exports by rivers based on a mass-balance approach for the land surface (watershed) and river system for year 2000 (Mayorga et al., 2010).

### D3 Inorganic P turnover rate

Soil solution P turnover (Km) were derived from isotopic exchange kinetic (IEK) experiments and aggregated on soil order level (Helfenstein et al., 2018).

### D4 Data-driven acid phosphatase activity

This global metric is an extrapolation of 126 European site observations of potential acid phosphatase activity from field samples by using back-propagation artificial network 12 environmental drivers as predictors (Sun et al., 2020).

## E: Evaluation datasets for leaf stoichiometry and resource use efficiencies

### E1 Gridded leaf N:P ratios

Global maps of leaf N and P concentrations datasets (Butler et al., 2017) were derived from global plant trait database Bayesian modeling. We used 100 sets of estimates of leaf N and P concentrations provided by Butler et al. (2017) to generate 10000 sets of global gridded leaf N:P ratio. Mean values and 25%~75% percentiles of leaf N:P ratios over those 10000 datasets were used to evaluate leaf N:P ratios by ORCHIDEE-CNP.

### E2 Remote-sensing fAPAP and PAR datasets

Global SeaWiFS Level-3 generates estimate of the Fraction of Absorbed Photosynthetically Active Radiation (FAPAR) over the land surface through an optimized index which values are computed on the basis of top of atmosphere bidirectional reflectance factor values, as well as information on the geometry of illumination and observation (Gobron et al., 2006a, b). This dataset is available for period 1997-2006 with spatial resolution of 0.01o.

### E3 Evapotranspiration and GPP from MTE

MTE-ET is data-oriented product derived by extrapolating the flux-tower observations from 4678 sites in space and time using climate data and remotely sensed data from 1982 to 2008 (Jung et al., 2011). This latter monthly GPP product now covers the period 1982-2011 at 0.5° spatial resolution, but many of the flux tower observations only cover much shorter periods.

## F: Evaluation datasets for LAI

The GIMMS-LAI3g data were derived from the Global Inventory Modeling and Mapping Studies (GIMMS) Normalized Differential Vegetation Index (NDVI) using the neural network algorithm (Zhu et al., 2013). This monthly dataset is available for period 1982-2015 with spatial resolution of 1/12 degree.

## G: Evaluation datasets for forest above-ground biomass

The GlobBiomass dataset of above-ground biomass (AGB) density (Fig. S2b) were generated by combining spaceborne Synthetic Aperture Radar (SAR), LiDAR (Light Detection And Ranging) and optical remote sensing observations. This data is available for the year of 2010 and with a spatial resolution of 100m. The global forest above-ground biomass density from GEOCARBON project (Fig. S2c) were generated from three existing datasets (Saatchi et al., 2011; Baccini et al., 2012; Santoro et al., 2015). The AGB estimate at 0.01o spatial resolution in the tropics was the weighted averages of data used in Saatchi et al. (2011) and Baccini et al. (2012), while that in the temperate and boreal forest corresponds to the biomass estimated in Santoro et al. (2015). These three datasets (i.e. Saatchi, Baccini and Santoro) were based on EO images acquired in different epochs (i.e. year 2000, 2007-2008 and year 2010). Simulated above-ground forest biomass carbon for the year of 2010 is compared with observation-based datasets.

## H: Evaluation datasets for SOC

### H1 Global Soil Dataset for use in Earth System Models (GSDE)

GSDE provides global gridded dataset of soil organic C (SOC) with a spatial resolution of 30 seconds and soil depth of 2.3m. This dataset is constructed based on the Soil Map of the World and various regional and national soil databases, including soil attribute data and soil maps (Shangguan et al., 2014).

### H2 SoilGrids

SoilGrids provides global gridded dataset of SOC with a spatial resolution of 250m and soil depth of 2m. This dataset is extrapolated from over 230 000 soil profile from the WoSIS database by using state-of-the-art machine learning methods (Hengl et al., 2017).

## I: sensitivity of GPP anomalies to anomalies of mean annual temperature (ST), annual precipitation (SP) and shortwave radiation (SR)

In this study, we estimated the response of GPP to climate variability by employing the multiple regression approach from Piao et al. (2013):

y=STxT+SPxP+SRxR+ε                                  (S4)

### where y is the de-trended anomaly of GPP, xT is the detrended mean annual temperature anomaly, xP is the de-trended annual precipitation anomaly, and xR is the detrended mean annual radiation anomaly. The fitted regression coefficients ST, SP, SR define the apparent GPP sensitivity to inter-annual variations in temperature, precipitation and radiation, respectively, and ɛ represents the residual error term. ORCHIDEE-CNP and ORCHIDEE used the same forcing data of meteorology from CRU-JRA-55. But BESS and MTE used climate datasets from CRU-NCEP. For analyzing the sensitivity of GPP anomaly to climates, we used CRU-JRA-55 for ORCHIDEE-CNP and ORCHIDEE and CRU-NCEP for BESS and MTE.

### S2. Influences of agricultural activities on N and P fluxes

The residence time of N and P in fertilized agricultural ecosystems is completely overridden by human input, so that model biases for those ecosystems should be related to how added nutrients are partitioned between their incorporation in biomass and losses, and soil P accumulation for P through sorption. Total N fertilizer input and harvest N for cropland by ORCHIDEE-CNP are 125 Tg N yr-1 and 75 Tg N yr-1 in 2010, which gives a high N use efficiency for crop biomass production equal to 0.6 (NUE; defined as the ratio of harvest N yield to total N inputs). Modelled N harvest by ORCHIDEE-CNP is very close with the FAOSTAT-based estimates of 74 Tg N yr-1 (Zhang et al., 2015), but our total N input prescribed from Zhang et al. (2015) is lower than FAOSTAT-based estimate of 174 Tg N yr-1. This difference is partly due to the different method to allocate fertilizer to crop and pasture areas by Zhang et al. (2015) (Conant et al., 2013; Lassaletta et al., 2014) and Lu and Tian (2017) (Monfreda et al., 2008). Further, biological nitrogen fixation (BNF) in croplands is about ~30 Tg N yr-1 according to Bodirsky et al., (2012) and 39 Tg N yr-1 for both cropland and pasture in Bouwman et al. (2013b), which was not accounted for in ORCHIDEE-CNP but was accounted in Zhang et al. (2015). A lower estimation in total N input by ORCHIDEE-CNP leads to a higher NUE compared to Zhang et al. (2015) (0.42).

Averaged total P input for cropland over 2002-2010 is estimated to be 20.4 Tg P yr-1, which is lower than the estimate by Lun et al. (2018) (25.5 Tg P yr-1). This is because we used a lower manure P input (4.2 Tg P yr-1) than Lun et al. (2018) (7.1 Tg P yr-1) and omitted sludge P input (1.4 Tg P yr-1 by Lun et al., 2018). Averaged modelled P harvest over 2002-2010 is 5.8 Tg P yr-1, which is also lower than that of Lun et al. (2018) (11.7 Tg P yr-1). The stoichiometry of crop harvest differs among crop types, while ORCHIDEE-CNP currently only distinguishes between three major crop types (maize, rice and wheat), which may explain the bias of P harvest whereas N harvest rates are in good agreement with independent estimates.

Modelled N and P leaching from croplands and pasture contributes 39% and 88% of global total N and P leaching in ORCHIDEE-CNP, respectively. The simulated global P leaching from managed land (3.3 Tg P yr-1) is lower than the estimation by Lun et al. (2018) of 5.4 Tg P yr-1) but higher than in Mekonnen and Hoekstra (2018) (0.6 Tg P yr-1). This large range of published estimates is due to the different methodologies and the assumptions of the ratio of P loss to P input (Liu et al., 2018). For example, Lun et al. (2018) assumed that a constant 12.5% of total P input was leached, while Mekonnen and Hoekstra (2018) estimated gridded P load to freshwater based on a much smaller erosion-runoff-leaching fraction (2.8% of total P input) for diffuse sources (Table S3).

### S3. Biome-scale nutrient budgets

In a model resolving nutrients like ORCHIDEE-CNP, as changes in climate and CO2 drive an increased land carbon storage, N and P tend to accumulate over time in biomass and soil carbon pools. We analyzed the modelled N and P storage changes for 4 managed and 10 natural biomes for the period of the 2000s from the ORCHIDEE-CNP output. We found a net N accumulation in natural biomes mainly fueled by BNF (Fig. S15a), except for boreal forests and natural grasslands where the contribution of N deposition is the dominant source of new N inputs. A net N accumulation of 14.4 Tg N yr-1 is found in pastures, whereas a net N loss of 6.5 Tg N yr-1 is found in croplands (Fig. S15a). N fertilizers application accounts for 20~33% of the total pasture N input, and dominates N inputs in cropland (80~95%, 149 Tg N yr-1 in total). Only 55~60% of N input from fertilizers application is harvested in crop biomass. The rest of these inputs being lost in gaseous form (45.7 Tg N yr-1) and from drainage and surface runoff leaching (20.7 Tg N yr-1).

We found a net P accumulation of 0.8 Tg P yr-1 in tropical and temperate forests, but a net P loss of -0.16 Tg P yr-1 in natural grasslands due to increased soil P fixation exceeding P deposition and weathering inputs. Boreal forests show a nearly balanced P budget (Fig. S15b). For tropical broad-leaved evergreen forests, inputs from deposition and weathering of primary and secondary minerals represent 55% and 45% of total P inputs, respectively. For other natural biomes, deposition dominates (>75%) the P inputs. P losses to soil P fixation are the major P loss pathway for all biomes (Fig. S15b).

A net P accumulation of 1.2 Tg P yr-1 is simulated for fertilized pasture and croplands, respectively (Fig. S15b). Manure fertilization dominates the total pasture P input (69%~93%) but mineral fertilizers dominates the total cropland P inputs (>95%). In croplands, 31% of cropland P input from fertilizers is harvested in crop biomass (5.9 Tg P yr-1), 36% is lost to soil P fixation (4.3 Tg P yr-1) and drainage and surface runoff (2.6 Tg P yr-1). The net P accumulation in pasture is mainly attributed to the omission of P output by grazing in ORCHIDEE-CNP.

### S4.  Biological nitrogen fixation

Biological nitrogen fixation (BNF) is the major natural input of N to terrestrial ecosystems (Vitousek et al., 2013) and its temperature dependence is critical for large-scale patterns of N limitations (Du et al., 2020) while its response to changing conditions critically affects the occurrence of N limitation under increasing CO2 (Goll et al., 2017b). Here we used BNF estimates from Sullivan et al. (2014) for tropical forests and a model-based global gridded BNP from Peng et al. (2019) to evaluate BNF in ORCHIDEE-CNP. A special attention is given to tropical forests where interactions between N and P are potentially controlling BNF (Houlton et al., 2008).

Sullivan et al. (2014) estimated symbiotic and free-living BNF of tropical forest sites based on acetylene reduction assays with nodules, soil and litter at 12 sites, using a spatial sampling method to generate unbiased estimates of mean symbiotic BNF independent of legume trees abundance, which more robustly captures the irregular distribution of nodules in the landscape. They found plot-level measurements of BNF rates which were five times lower than used in previous empirical upscaling to the tropical forest biomes (Cleveland et al. 1999; Wang and Houlton, 2009). Recalibrating the empirical relationship between BNF and either NPP or evapotranspiration from Cleveland et al. (1999) and Wang and Houlton et al. (2009) based on their plot level estimates of BNF, Sullivan et al. (2014) upscaled plot-level BNF to all tropical forests to 4.9~6.3 kg N ha-1 yr-1 and 3.7~7.8 kg N ha-1 yr-1 for first and third quantiles, respectively. Peng et al. (2019) developed a global BNF model constrained by observed C:N ratios of various plant pools (Sect. S1D1 in the supplement), and they simulated a higher BNF for tropical forests than Sullivan et al. (2014) with an interquartile range (IQR) of 14~43 kg N ha-1 yr-1. BNF by ORCHIDEE-CNP in tropical forests is comparable in median value (5.6 kgN ha-1 yr-1) but has a wider IQR of 2.5-7.8 kg N ha-1 yr-1 than the upscaled estimation according to Sullivan et al. (2014) (Fig. S16), but is lower than Peng et al. (2019)’s estimate, even lower than their first quartile. For all biomes, ORCHIDEE-CNP simulated a higher total BNF (153 Tg N yr-1) than Peng et al. (2019) (116 Tg N yr-1) and a roughly comparable spatial range (IQR = 3.1-18 kg N ha-1 yr-1 compared to 0.23-15 kg N ha-1 yr-1) (Fig. S16). Specifically, ORCHIDEE-CNP simulated a tropical BNF which is close to estimates derived from measurements, but a higher global BNF of 153 Tg N yr-1 than previous estimates (40-128 Tg N yr-1) (Galloway et al., 2004; Vitousek et al., 2013), primarily due to an overestimation of extratropical BNF rates (Fig. S16). The latter is likely related to the omission of a direct temperature control on BNF in ORCHIDEE-CNP which was shown to be able to explain low BNF rates at higher latitudes (Houlton et al., 2008).

### S5. N and P load rates to river catchment scale

We used the export of N and P from terrestrial systems from the Global Nutrient Export from Water-Sheds(GlobalNEWS2) model (Table 1). This model gives N and P loading to rivers by surface runoff and drainage based on a mass-balance approach (Mayorga et al., 2010). As ORCHIDEE-CNP lacks a representation of nutrient soil infiltration and groundwater networks, we had to approximate N and P load from ORCHIDEE output variables. Since most of dissolved N in groundwater enters the river network via drainage (Bouwman et al., 2013a), we extracted annual N exported via drainage and surface runoff from the ORCHIDEE-CNP output to be compared with the GlobalNEWS2 load rates (Nload) on catchments scale:

Nload=N runoff+N drainage ,						    (S1)

where N runoff is the simulated annual N flux lost by surface runoff, and N drainage the annual N flux lost by drainage both in kgN km-2 yr-1.

Dissolved P which drains in deeper soil layers (Pdra) is mostly adsorbed by soil minerals and only a minor fraction enters the groundwater. Thus, the annual soil P loss via surface runoff (kg P km-2 yr-1) from the ORCHIDEE-CNP output were extracted, and were compared with the GlobalNEWS2 load rates (𝑃load):

Pload=Prunoff ,							               (S2)

ORCHIDEE-CNP simulated a global Nload calculated over the period 2002-2010 of 55 Tg N yr-1 with Nload from natural biomes being 35 Tg N yr-1, close to the estimate by GOLUM-CNP (38 Tg N yr-1) but higher than one by GlobalNEWS2 (28 Tg N yr-1 in year 2000). Simulated Nload from managed land, namely croplands and pastures, is 21 Tg N yr-1, about half the estimate from GlobalNEWS2 (40 Tg N yr-1 in year 2000; Mayorga et al., 2010). This underestimation by ORCHIDEE-CNP is likely due to the lower manure N fertilizer scenario, as part of the model forcing, than used in GlobalNEWS2 (Fig. S21), due to the omission of point sources such as livestock farms and sewage systems. On catchment scale, Nload in the year 2000 simulated by ORCHIDEE-CNP have a comparable range (0~2500 kg N km-2 yr-1) but generally lower values than GlobalNEWS2 (Fig. S17a to c), excepted for some basins dominated by Oxisols such as the Amazon (Fig. S18). The overestimation of Nload in some tropical basins dominated by Oxisols soils (e.g. Amazon) (Fig. S17) is mainly attributed to our assumptions regarding the sum of Nrunoff and Ndra to approximate Nload (Eq. S1). Tropical basins have strong N denitrification losses from the groundwater as a result of large flux and high N concentration of groundwater (Bouwman et al., 2013a), thereby a part of the N is lost before the water enters the river (i.e. Ndra). Denitrification losses from groundwater are not accounted for in ORCHIDEE-CNP. In addition, ORCHIDEE-CNP applies a globally uniform ammonium sorption capacity of soils, calibrated to a limited number of soil measurements (Zaehle and Friend, 2010) which might not be applicable to highly weathered and low pH soils in the tropics.

Global total Pload* (Pload* =  Pdra+ Prunoff) is estimated at 3.8 Tg P yr-1 averaged over the period 2002-2010 in ORCHIDEE-CNP. Natural biomes account for only ~10% of global total Pload* (0.39 Tg P yr-1), about an order of magnitude less than in GOLUM-CNP (Table S4). This lower total Pload* for natural biomes mainly occurs in forest ecosystems (Fig. S17) which can be mainly attributed to the low substrate availability (i.e. dissolved P concentration) due to strong P immobilization into biomass and SOM for forests (Fig. S1a, g) in ORCHIDEE-CNP. Croplands and pastures account for most of the global total Pload* with a value of 3.3 Tg P yr-1 which is rather close to the estimate of 4 Tg P yr-1 by Bouwman et al. (2013b), but lower than the 5.4 Tg P yr-1 reported by Lun et al. (2018) and higher than in Mekonnen and Hoekstra (2018) (0.6 Tg P yr-1) (Table S3). ORCHIDEE-CNP cannot capture the high P losses from extremely high livestock densities in small areas where soil P retention capacities have been reached. Besides, Lun et al. (2018) considered sludge inputs (1.4 Tg P yr-1 in year 2000), which are omitted in ORCHIDEE-CNP. The simulated values of Pload (Eq. S2) at catchment scale are in the same order of magnitude than estimates from GlobalNEWS2 (Mayorga et al., 2010). However, ORCHIDEE-CNP simulates generally lower Pload over the Amazon and Central African basins compared to GlobalNEWS2 (Mayorga et al., 2010) (Fig. S17d, e). This underestimation can be partly attributed to the strong P fixation capacity of soil (i.e. occlusion) in tropical regions in ORCHIDEE-CNP, which is related to a fast turnover of soil inorganic P (Sect. S6 in the supplement). Note that Pdra is small but non-negligible in surface and shallow subsurface runoff (Mayorga et al., 2010), and ignoring this flux for Pload may lead to our underestimation of Pload (Eq. S2). Simulated Pload is also higher than GlobalNEWS2 in European catchments, where pasture is widespread (Fig. 17d, e), which can be attributed to either a less effective soil P sorption capacity or the omission of grazing which enhances the mineralization of organic P and as a consequence inorganic P losses.

### S6. Soil solution inorganic phosphorus turnover rate

The simulated soil P fluxes cannot be evaluated against direct observations (Frossard et al., 2011). Here we used pool turnover times instead of fluxes to evaluate P dynamics, as the turnover of P in the soil solution is an important component of P bioavailability in soil (Helfenstein et al., 2018). We compared soil solution P turnover in ORCHIDEE-CNP with empirical data from isotope exchange kinetic (IEK) experiments (Fardeau et al. 1991; Frossard et al. 2011). IEK experiments measure the exchange rate of inorganic P between soil solution and the soil solid phase (Km, unit: min-1) in laboratory conditions, omitting biological processes. The inorganic exchange processes captured in Km dominate short-term P fluxes.

For a straightforward comparison (excluding biological uptake process from model output to match the experimental method used for measuring), we diagnosed Km (unit: min-1) in the simulations from the net exchange between dissolved and sorbed labile P:

Km=Pdissolved∆Psorbed-Fdss , 							        (S3)

Where Pdissolved is the labile P in soil solution (g P m-2) and ∆Psorbed the change in labile P adsorbed (g P m-2 t-1) with t=30 minutes (model time step) corrected for the loss term of Psorbed related to strongly sorption. In ORCHIDEE-CNP, the exchange between Pdissolved and Psorbed (Fdss) is calculated using a Freundlich adsorption isotherm by assuming a chemical equilibrium between both pools is reached on the model time step (Goll et al., 2018).

We used data from a global compilation of empirical Km data from 218 different soil samples, mainly from Europe and North America but also covering Asian and Africa (Helfenstein et al., 2018). We found that simulated Kmare of similar magnitude with median value of 24~468 min-1 across soil orders compared to those empirical data with simulated 5~990 min-1 (Fig. S19). However, the turnover rates for different soil types did not match empirical data. The model overestimated Km for Alfisols, Aridisols, Entisols, Mollisols, Spodosols, Ultisols, and underestimated Km for Oxisols and Vertisols.

ORCHIDEE-CNP cannot capture variations in Km among different soil orders as derived from measurements (Helfenstein et al., 2018), that is because that ORCHIDEE-CNP distinguishes only between three classes of soils with respect to parameter controlling inorganic P sorption dynamics: Oxisols, Molisols and other soils (Table S5; Goll et al., 2017a). The difference in biases of Km between Oxisols and other soil suborders suggests that our parameterization of sorption is the primary cause for the model bias, as all other processes affecting dissolved P are not a function of soil type. In the P-enabled LSMs, inorganic P processes operating on longer timescale (occlusion, strong sorption) are only simply represented (Wang et al., 2010; Yang et al., 2014; Goll et al., 2017b). This processes in LSMs is primarily based on calibration rather than data driven, which remain a large source of uncertainty regarding changes in P availability under elevated CO2 (Goll et al., 2012). Recent advances in the quantification of inorganic soil P turnover (Helfenstein et al., 2020) should be used to improve this part of models.

### S7. P biochemical mineralization from phosphatase enzymes

Biochemical phosphorus mineralization (BPM) mediated by extracellular phosphatase enzymes produced plants and microbes through cleaving P out of organic matter, is a potentially significant pathway for mineralizing P (Wang et al., 2007). ORCHIDEE-CNP includes an empirical parameterization of BPM which accounted for an enhanced mineralization when plants experience suboptimal P-to-N availabilities, as an approximation of the stoichiometric status of the whole ecosystem, including the effect of substrate availability on mineralization (McGill and Cole, 1981) (see Eq. 18 in Goll et al., 2017b). Potential phosphatase activity (Pases) can be measured in the lab from soil samples, and serves as a surrogate for the lack of direct BPM measurements in real-world soil conditions (Sun et al., 2020). We compare the global pattern of simulated P mineralization fluxes in ORCHIDEE-CNP with the map of Pases extrapolated from site measurements (Margalef et al., 2017) produced by Sun et al. (2020). The modelled BPM is higher in tropical regions than in desert and boreal regions, consistent with the Pases pattern (Fig. S20), although the scarcity of tropical measurements makes the high values observed in these regions more uncertain (Sun et al., 2020).

Global organic P mineralization fluxes were evaluated against potential phosphatase activity (Pases) data (Margalef et al., 2017; Sun et al., 2020), and actual bulk P mineralization measurements based on radioisotopic approaches (Bünemann, 2015; Wanek et al., 2019), to our knowledge the first attempt to evaluate a model for this process. ORCHIDEE-CNP can capture the general global pattern of Pases (Fig. S20b), which gives us some confidence that large-scale differences with respect to the decoupling of P mineralization from the mineralization of C and N can be roughly captured by the model.

Direct measurements of bulk organic P mineralization using radio-isotopic approaches are only available for few soils. If up-scaled to yearly rates, measured organic P mineralization rates range between 101 to 103 g P m-2 yr-1 (Bünemann, 2015; Wanek et al., 2019) which are orders of magnitude higher than simulated by ORCHIDEE-CNP (0.53 ± 0.48 g P m-2 yr-1; Fig. S20a). It has to be noted that lab measurements of P mineralization rates and potential phosphatase activity are conducted under optimal conditions, whereas in the field, temperature and water hamper enzyme activity for large parts of the year. This illustrates that the evaluation of model performance in representing soil organic P mineralization remains a challenge since quantification of apparent rates under field conditions are currently missing.

References

Baccini, A., Goetz, S., Walker, W., Laporte, N., Sun, M., Sulla-Menashe, D., Hackler, J., Beck, P., Dubayah, R., Friedl, M., Samanta, S., and Houghton, R.: Estimated carbon dioxide emissions from tropical deforestation improved by carbon-density maps, Nat. Clim. Chang., 2, 182-185, http://doi.org/10.1038/nclimate1354, 2012.

Bodirsky, B., Popp, A., Weindl, I., Dietrich, J., Rolinski, S., Scheiffele, L., Schmitz, C., and Lotze-Campen, H.: N2O emissions from the global agricultural nitrogen cycle - current state and future scenarios, Biogeosciences, 9, 4169-4197, http://doi.org/10.5194/bg-9-4169-2012, 2012.

Bouwman, A., Beusen, A., Griffioen, J., Van Groenigen, J., Hefting, M., Oenema, O., Van Puijenbroek, P., Seitzinger, S., Slomp, C., and Stehfest, E.: Global trends and uncertainties in terrestrial denitrification and N2O emissions, Philos. Trans. R. Soc. B-Biol. Sci., 368, http://doi.org/10.1098/rstb.2013.0112, 2013a.

Bouwman, L., Goldewijk, K. K., Van Der Hoek, K. W., Beusen, A. H., Van Vuuren, D. P., Willems, J., Rufino, M. C., and Stehfest, E.: Exploring global changes in nitrogen and phosphorus cycles in agriculture induced by livestock production over the 1900–2050 period, Proc. Natl. Acad. Sci. U. S. A., 110, 20882-20887, http://doi.org/10.1073/pnas.1012878108, 2013b.

Bünemann, E. K.: Assessment of gross and net mineralization rates of soil organic phosphorus–A review, Soil Biol. Biochem., 89, 82-98, http://doi.org/10.1016/j.soilbio.2015.06.026, 2015.

Butler, E. E., Datta, A., Flores-Moreno, H., Chen, M., Wythers, K. R., Fazayeli, F., Banerjee, A., Atkin, O. K., Kattge, J., Amiaud, B., Blonder, B., Boenisch, G., Bond-Lamberty, B., Brown, K. A., Byun, C., Campetella, G., Cerabolini, B. E. L., Cornelissen, J. H. C., Craine, J. M., Craven, D., de Vries, F. T., Díaz, S., Domingues, T. F., Forey, E., González-Melo, A., Gross, N., Han, W., Hattingh, W. N., Hickler, T., Jansen, S., Kramer, K., Kraft, N. J. B., Kurokawa, H., Laughlin, D. C., Meir, P., Minden, V., Niinemets, Ü., Onoda, Y., Peñuelas, J., Read, Q., Sack, L., Schamp, B., Soudzilovskaia, N. A., Spasojevic, M. J., Sosinski, E., Thornton, P. E., Valladares, F., van Bodegom, P. M., Williams, M., Wirth, C., and Reich, P. B.: Mapping local and global variability in plant trait distributions, Proc. Natl. Acad. Sci. U. S. A., 114, E10937-E10946, http://doi.org/10.1073/pnas.1708984114, 2017.

Chevallier, F., Fisher, M., Peylin, P., Serrar, S., Bousquet, P., Bréon, F. M., Chédin, A., and Ciais, P.: Inferring CO2 sources and sinks from satellite observations: Method and application to TOVS data, J. Geophys. Res.-Atmos., 110, http://doi.org/10.1029/2005JD006390, 2005.

Cleveland, C. C., Townsend, A. R., Schimel, D. S., Fisher, H., Howarth, R. W., Hedin, L. O., Perakis, S. S., Latty, E. F., Fischer, J. C. V., Elseroad, A., and Wasson, M. F.: Global patterns of terrestrial biological nitrogen (N2) fixation in natural ecosystems, Global biogeochem. cycles, 13(2), 623-645, https://doi.org/10.1029/1999GB900014,1999.

Conant, R. T., Berdanier, A. B., and Grace, P. R.: Patterns and trends in nitrogen use and nitrogen recovery efficiency in world agriculture, Global Biogeochem. Cycles, 27, 558-566, http://doi.org/10.1002/gbc.20053, 2013.

Du, E., Terrer, C., Pellegrini, A. F., Ahlström, A., van Lissa, C. J., Zhao, X., Xia, N., Wu, X., and Jackson, R. B.: Global patterns of terrestrial nitrogen and phosphorus limitation, Nat. Geosci., 13, 221-226, http://doi.org/10.1038/s41561-019-0530-4, 2020.

Fardeau, J., Morel, C., and Boniface, R.: Phosphate ion transfer from soil to soil solution: kinetic parameters [available P, phosphate flow], Agronomie, http://doi.org/10.1051/agro:19910909, 1991.

Frossard, E., Achat, D. L., Bernasconi, S. M., Bünemann, E. K., Fardeau, J.-C., Jansa, J., Morel, C., Rabeharisoa, L., Randriamanantsoa, L., Sinaj, S., Tamburini, F., and Oberson, A.: The use of tracers to investigate phosphate cycling in soil–plant systems, in: Phosphorus in Action, Springer, 59-91, 2011.

Gobron, N., Pinty, B., Aussedat, O., Chen, J., Cohen, W. B., Fensholt, R., Gond, V., Huemmrich, K. F., Lavergne, T., Mélin, F., Privette, J. L., Sandholt, I., Taberner, M., Turner, D. P., Verstraete, M. M., and Widlowski, J.: Evaluation of fraction of absorbed photosynthetically active radiation products for different canopy radiation transfer regimes: Methodology and results using Joint Research Center products derived from SeaWiFS against ground-based estimations, J. Geophys. Res.-Atmos, 111, D13110, http://doi.org/10.1029/2005JD006511, 2006a.

Gobron, N., Pinty, B., Taberner, M., Mélin, F., Verstraete, M., and Widlowski, J.-L.: Monitoring the photosynthetic activity of vegetation from remote sensing data, Adv. Space Res., 38, 2196-2202, http://doi.org/10.1016/j.asr.2003.07.079, 2006b.

Goll, D. S., Brovkin, V., Parida, B., Reick, C. H., Kattge, J., Reich, P. B., Van Bodegom, P., and Niinemets, Ü.: Nutrient limitation reduces land carbon uptake in simulations with a model of combined carbon, nitrogen and phosphorus cycling, Biogeosciences, 9, 3547-3569, http://doi.org/10.5194/bg-9-3547-2012, 2012.

Goll, D., Vuichard, N., Maignan, F., Jornet-Puig, A., Sardans, J., Violette, A., Peng, S., Sun, Y., Kvakic, M., and Guimberteau, M.: A representation of the phosphorus cycle for ORCHIDEE (revision 4520), Geosci. Model Dev., 10, 3745-3770, http://doi.org/10.5194/gmd-10-3745-2017, 2017a.

Galloway, J. N., Dentener, F. J., Capone, D. G., Boyer, E. W., Howarth, R. W., Seitzinger, S. P., Asner, G. P., Cleveland, C. C., Green, P., Holland, E. A. Karl, D. M., Michaels, A. F., Porter, J. H., Townsend, A. R., Vöosmarty, C. J.: Nitrogen cycles: past, present, and future, Biogeochemistry, 70, 153-226, http://doi.org/10.1007/s10533-004-0370-0, 2004.

Goll, D. S., Winkler, A. J., Raddatz, T., Dong, N., Prentice, I. C., Ciais, P., and Brovkin, V.: Carbon-nitrogen interactions in idealized simulations with JSBACH (version 3.10), Geosci. Model Dev., 10, 2009-2030, http://doi.org/10.5194/gmd-10-2009-2017, 2017b.

Goll, D. S., Joetzjer, E., Huang, M., and Ciais, P.: Low phosphorus availability decreases susceptibility of tropical primary productivity to droughts, Geophys. Res. Lett., 45, 8231-8240, http://doi.org/10.1029/2018GL077736, 2018.

Helfenstein, J., Jegminat, J., McLaren, T. I., and Frossard, E.: Soil solution phosphorus turnover: derivation, interpretation, and insights from a global compilation of isotope exchange kinetic studies, Biogeosciences, 15, 105-114, http://doi.org/10.5194/bg-15-105-2018, 2018.

Helfenstein, J., Pistocchi, C., Oberson, A., Tamburini, F., Goll, D. S., and Frossard, E.: Estimates of mean residence times of phosphorus in commonly considered inorganic soil phosphorus pools, Biogeosciences, 17, 441-454, http://doi.org/10.5194/bg-17-441-2020, 2020.

Hengl, T., de Jesus, J. M., MacMillan, R. A., Batjes, N. H., Heuvelink, G. B., Ribeiro, E., Samuel-Rosa, A., Kempen, B., Leenaars, J. G., Walsh, M. G. Gonzalez, M. R.: SoilGrids1km—global soil information based on automated mapping, PLoS One, 9, http://doi.org/10.1371/journal.pone.0105992, 2014.

Houlton, B. Z., Wang, Y.-P., Vitousek, P. M., and Field, C. B.: A unifying framework for dinitrogen fixation in the terrestrial biosphere, Nature, 454, 327-330, http://doi.org/10.1038/nature07028, 2008.

Jiang, M., Caldararu, S., Zaehle, S., Ellsworth, D. S., and Medlyn, B. E.: Towards a more physiological representation of vegetation phosphorus processes in land surface models, New Phytol., 222, 1223-1229, http://doi.org/10.1111/nph.15688, 2019.

Jung, M., Reichstein, M., and Bondeau, A.: Towards global empirical upscaling of FLUXNET eddy covariance observations: validation of a model tree ensemble approach using a biosphere model, Biogeosciences, http://doi.org/10.5194/bg-6-2001-2009, 2009.

Jung, M., Reichstein, M., Margolis, H. A., Cescatti, A., Richardson, A. D., Arain, M. A., Arneth, A., Bernhofer, C., Bonal, D., Chen, J., Damiano, G., Gobron, N., Kiely, G., Kutsch, W., Lasslop, G., Law, B. E., Lindroth, A., Merbold, L., Montagnani, L., Moors, E. J., Papale, D., Sottocornola, M., Vaccari, F., and Williams, C.: Global patterns of land-atmosphere fluxes of carbon dioxide, la- tent heat, and sensible heat derived from eddy covariance, satel- lite, and meteorological observations, J. Geophys. Res.-Biogeo., 116, http://doi.org/10.1029/2010JG001566, 2011.

Krinner, G., Viovy, N., de Noblet‐Ducoudré, N., Ogée, J., Polcher, J., Friedlingstein, P., Ciais, P., Sitch, S., and Prentice, I. C.: A dynamic global vegetation model for studies of the coupled atmosphere‐biosphere system, Glob. Biogeochem. Cycle, 19, http://doi.org/10.1029/2003GB002199, 2005.

Lassaletta, L., Billen, G., Grizzetti, B., Anglade, J., and Garnier, J.: 50 year trends in nitrogen use efficiency of world cropping systems: the relationship between yield and nitrogen input to cropland, Environ. Res. Lett., 9, 105011, http://doi.org/10.1088/1748-9326/9/10/105011, 2014.

Liu, W., Yang, H., Ciais, P., Stamm, C., Zhao, X., Williams, J. R., Abbaspour, K. C., and Schulin, R.: Integrative Crop‐Soil‐Management Modeling to Assess Global Phosphorus Losses from Major Crop Cultivations, Glob. Biogeochem. Cycle, 32, 1074-1086, http://doi.org/10.1029/2017GB005849, 2018.

Lu, C. C., and Tian, H.: Global nitrogen and phosphorus fertilizer use for agriculture production in the past half century: shifted hot spots and nutrient imbalance, Earth Syst. Sci. Data, 9, 181, http://doi.org/10.5194/essd-9-181-2017, 2017.

Lun, F., Liu, J., Ciais, P., Nesme, T., Chang, J., Wang, R., Goll, D., Sardans, J., Peñuelas, J., and Obersteiner, M.: Global and regional phosphorus budgets in agricultural systems and their implications for phosphorus-use efficiency, Earth Syst. Sci. Data, 10, 1-18, http://doi.org/10.5194/essd-10-1-2018, 2018.

Margalef, O., Sardans, J., Fernández-Martínez, M., Molowny-Horas, R., Janssens, I., Ciais, P., Goll, D., Richter, A., Obersteiner, M., Asensio, D., and Peñuelas, J.: Global patterns of phosphatase activity in natural soils, Sci. Rep., 7, 1-13, http://doi.org/10.1038/s41598-017-01418-8, 2017.

Mayorga, E., Seitzinger, S. P., Harrison, J. A., Dumont, E., Beusen, A. H., Bouwman, A., Fekete, B. M., Kroeze, C., and Van Drecht, G.: Global nutrient export from WaterSheds 2 (NEWS 2): model development and implementation, Environ. Modell. Softw., 25, 837-853, http://doi.org/10.1016/j.envsoft.2010.01.007, 2010.

Mekonnen, M. M., and Hoekstra, A. Y.: Global anthropogenic phosphorus loads to freshwater and associated grey water footprints and water pollution levels: A high‐resolution global study, Water Resour. Res., 54, 345-358, http://doi.org/10.1002/2017WR020448, 2018.

McGill, W., and Cole, C.: Comparative aspects of cycling of organic C, N, S and P through soil organic matter, Geoderma, 26, 267-286, http://doi.org/10.1016/0016-7061(81)90024-0, 1981.

McGroddy, M. E., Daufresne, T., and Hedin, L. O.: Scaling of C: N: P stoichiometry in forests worldwide: Implications of terrestrial redfield‐type ratios, Ecology, 85, 2390-2401, http://doi.org/10.1890/03-0351, 2004.

Monfreda, C., Ramankutty, N., and Foley, J. A.: Farming the planet: 2. Geographic distribution of crop areas, yields, physiological types, and net primary production in the year 2000, Glob. Biogeochem. Cycle, 22, http://doi.org/10.1029/2007GB002947, 2008.

Peng, J., Wang, Y. P., Houlton, B. Z., Dan, L., Pak, B., and Tang, X.: Global carbon sequestration is highly sensitive to model‐based formulations of nitrogen fixation, Glob. Biogeochem. Cycle, http://doi.org/10.1029/2019GB006296, 2019.

Piao, S., Sitch, S., Ciais, P., Friedlingstein, P., Peylin, P., Wang, X., Ahlström, A., Anav, A., Canadell, J. G., Cong, N., Huntingford, C., Jung, M., Levis, S., Levy, P. E., Li, J., Lin, X., Lomas, M. R., Lu, M., Luo, Y., Ma, Y., Myneni, R. B., Poulter, B., Sun, Z., Wang, T., Viovy, N., Zaehle, S., and Zeng, N.: Evaluation of terrestrial carbon cycle models for their response to climate variability and to CO2 trends, Global Change Biol., 19, 2117– 2132, http://doi.org/10.1111/gcb.12187, 2013.

Rödenbeck, C., Houweling, S., Gloor, M., and Heimann, M.: CO2 flux history 1982-2001 inferred from atmospheric data using a global inversion of atmospheric transport, Atmos. Chem. Phys., 3, 1919-1964, 2003.

Running, S. W., Nemani, R. R., Heinsch, F. A., Zhao, M., Reeves, M., and Hashimoto, H.: A continuous satellite-derived measure of global terrestrial primary production, BioScience, 54, 547-560, http://doi.org/10.1641/0006-3568(2004)054[0547:ACSMOG]2.0.CO;2, 2004.

Saatchi, S. S., Harris, N. L., Brown, S., Lefsky, M., Mitchard, E. T., Salas, W., Zutta, B. R., Buermann, W., Lewis, S. L., Hagen, S., Petrova, S., White, L., Silman, M., Morel, A.: Benchmark map of forest carbon stocks in tropical regions across three continents, Proc. Natl. Acad. Sci. U. S. A., 108, 9899-9904, http://doi.org/10.1073/pnas.1019576108, 2011.

Santoro, M., Eriksson, L. E., and Fransson, J. E.: Reviewing ALOS PALSAR backscatter observations for stem volume retrieval in Swedish forest, Remote Sens., 7, 4290-4317, http://doi.org/10.3390/rs70404290, 2015.

Shangguan, W., Dai, Y., Duan, Q., Liu, B., Yuan, H.: A global soil data set for earth system modeling, J. Adv. Model Earth Syst., 6.1, 249-263, https://doi.org/10.1002/2013MS000293, 2014.

Smith, W. K., Reed, S. C., Cleveland, C. C., Ballantyne, A. P., Anderegg, W. R., Wieder, W. R., Liu, Y. Y., and Running, S. W.: Large divergence of satellite and Earth system model estimates of global terrestrial CO2 fertilization, Nat. Clim. Chang., 6, 306-310, http://doi.org/10.1038/NCLIMATE2879, 2016.

Stephens, B. B., Gurney, K. R., Tans, P. P., Sweeney, C., Pe- ters, W., Bruhwiler, L., Ciais, P., Ramonet, M., Bousquet, P., Nakazawa, T., Aoki, S., Machida, T., Inoue, G., Vinnichenko, N., Lloyd, J., Jordan, A., Heimann, M., Shibistova, O., Langen- felds, R. L., Steele, L. P., Francey, R. J., and Denning, A. S.: Weak northern and strong tropical land carbon uptake from ver- tical profiles of atmospheric CO2, Science, 316, 1732–1735, , http://doi.org/10.1126/science.1137004, 2007.

Sullivan, B. W., Smith, W. K., Townsend, A. R., Nasto, M. K., Reed, S. C., Chazdon, R. L., and Cleveland, C. C.: Spatially robust estimates of biological nitrogen (N) fixation imply substantial human alteration of the tropical N cycle, Proc. Natl. Acad. Sci. U. S. A., 111, 8101-8106, http://doi.org/10.1073/pnas.1320646111, 2014.

Sun, Y., Goll, D. S., Ciais, P., Peng, S., Margalef, O., Asensio, D., Sardans, J., and Peñuelas, J.: Spatial pattern and environmental drivers of acid phosphatase activity in Europe, Front. Big Data, http://doi.org/10.3389/fdata.2019.00051, 2020.

Tum, M., Zeidler, J., Günther, K. P., and Esch, T.: Global NPP and straw bioenergy trends for 2000–2014, Biomass Bioenerg., 90, 230-236, http://doi.org/10.1016/j.biombioe.2016.03.040, 2016.

Turner, D. P., Ritts, W. D., Cohen, W. B., Gower, S. T., Running, S. W., Zhao, M., Costa, M. H., Kirschbaum, A. A., Ham, J. M., Saleska, S. R., and Ahl, D.: Evaluation of MODIS NPP and GPP products across multiple biomes, Remote Sens. Environ., 102, 282-292, http://doi.org/10.1016/j.rse.2006.02.017, 2006.

van der Laan-Luijkx IT, van der Velde IR, van der Veen E, Tsuruta A, Stanislawska K, Babenhauserheide A, Zhang HF, Liu Y, He W, Chen H, Masarie KA, Krol MC, and Peters W: The CarbonTracker Data Assimilation Shell (CTDAS) v1. 0: implementation and global carbon balance 2001-2015, Geosci. Model Dev., 10, 2785-2800, http://doi.org/10.5194/gmd-10-2785-2017, 2017.

Vitousek, P. M., Menge, D. N., Reed, S. C., and Cleveland, C. C.: Biological nitrogen fixation: rates, patterns and ecological controls in terrestrial ecosystems, Philos. Trans. R. Soc. B-Biol. Sci., 368, 20130119, http://doi.org/10.1098/rstb.2013.0119, 2013.

Vuichard, N., Messina, P., Luyssaert, S., Guenet, B., Zaehle, S., Ghattas, J., Bastrikov, V., and Peylin, P.: Accounting for carbon and nitrogen interactions in the global terrestrial ecosystem model ORCHIDEE (trunk version, rev 4999): multi-scale evaluation of gross primary production, Geosci. Model Dev., 12, 4751-4779, http://doi.org/10.5194/gmd-12-4751-2019, 2019.

Wanek, W., Zezula, D., Wasner, D., Mooshammer, M., and Prommer, J.: A novel isotope pool dilution approach to quantify gross rates of key abiotic and biological processes in the soil phosphorus cycle, Biogeosciences, 16, 3047-3068, http://doi.org/10.5194/bg-16-3047-2019, 2019.

Wang, Y., Law, R., and Pak, B.: A global model of carbon, nitrogen and phosphorus cycles for the terrestrial biosphere, Biogeosciences, 7, http://doi.org/10.5194/bg-7-2261-2010, 2010.

Wang, Y. P., Houlton, B., and Field, C.: A model of biogeochemical cycles of carbon, nitrogen, and phosphorus including symbiotic nitrogen fixation and phosphatase production, Glob. Biogeochem. Cycle, 21, http://doi.org/10.1029/2006GB002797, 2007.

Wang, Y. P., and Houlton, B. Z.: Nitrogen constraints on terrestrial carbon uptake: Implications for the global carbon‐climate feedback, Geophys. Res. Lett., 36, http://doi.org/10.1029/2009GL041009, 2009.

Wißkirchen, K., Tum, M., Günther, K., Niklaus, M., Eisfelder, C., and Knorr, W.: Quantifying the carbon uptake by vegetation for Europe on a 1km 2 resolution using a remote sensing driven vegetation model, Geosci. Model Dev., 6, http://doi.org/10.5194/gmd-6-1623-2013, 2013.

Yang, X., Thornton, P., Ricciuto, D., and Post, W.: The role of phosphorus dynamics in tropical forests--a modeling study using CLM-CNP, Biogeosciences, 11, http://doi.org/10.5194/bg-11-1667-2014, 2014.

Zaehle, S., and Friend, A.: Carbon and nitrogen cycle dynamics in the O‐CN land surface model: 1. Model description, site‐scale evaluation, and sensitivity to parameter estimates, Glob. Biogeochem. Cycle, 24, http://doi.org/10.1029/2009GB003521, 2010.

Zhang, X., Davidson, E. A., Mauzerall, D. L., Searchinger, T. D., Dumas, P., and Shen, Y.: Managing nitrogen for sustainable development, Nature, 528, 51-59, http://doi.org/10.1038/nature15743, 2015.

Zhao, M., Heinsch, F. A., Nemani, R. R., and Running, S. W.: Improvements of the MODIS terrestrial gross and net primary production global data set, Remote Sens. Environ., 95, 164-176, http://doi.org/10.1016/j.rse.2004.12.011, 2005.

Zhu, Z., Bi, J., Pan, Y., Ganguly, S., Anav, A., Xu, L., Samanta, A., Piao, S., Nemani, R. R., and Myneni, R. B.: Global data sets of vegetation leaf area index (LAI) 3g and fraction of photosynthetically active radiation (FPAR) 3g derived from global inventory modeling and mapping studies (GIMMS) normalized difference vegetation index (NDVI3g) for the period 1981 to 2011, Remote Sens. , 5, 927-948, http://doi.org/10.3390/rs5020927, 2013.

Table S1 Plant functional type (PFT) specific leaf N:P ratio (gN g-1P ) for ORCHIDEE v1.1 and v1.2: tropical evergreen broadleaf forest (TREBF), tropical raingreen broadleaf forest (TRDBF), temperate evergreen needleleaf forest (TEENF), temperate evergreen broadleaf forest (TEEBF), temperate summergreen broadleaf forest (TEDBF), boreal evergreen needleleaf forest (BOENF), boreal summergreen broadleaf forest (BODBF), boreal summergreen needleleaf forest (BODNF), C3 grassland (C3G), C4 grassland (C4G), C3 cropland (C3C) and C4 cropland (C4C).

|  | TREBF | TRDBF | TRENF | TREBF | TRDBF | BOENF | BODNF | BODBF | C3G | C4G | C3C | C4C | Sources |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| npleaf,min for v1.1 | 16.68 | 16.68 | 8.34 | 10.84 | 10.84 | 8.34 | 10.84 | 10.84 | 10.84 | 10.84 | - | - | McGroddy et al. (2004) |
| npleaf,max for v1.1 | 22.57 | 22.57 | 11.29 | 14.67 | 14.67 | 11.29 | 14.67 | 14.67 | 14.67 | 14.67 | - | - | McGroddy et al. (2004) |
| npleaf,min for v1.2 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |
| npleaf,max for v1.2 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |  |

Table S2 Total GPP and NPP (PgC yr-1) simulated by ORCHIDEE-CNP, ORCHIDEE and from data-driven products averaged over 2001-2010.

|  |  | GPP | Sources |  |  | NPP |  | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ORCHIDEE-CNP | 119 |  | This study | ORCHIDEE-CNP |  |  | 48 | This study |
| ORCHIDEE | 140 |  | Krinner et al., 2005 | ORCHIDEE |  |  | 60 | Krinner et al., 2005 |
| MODIS | 110±23 |  | Turner et al., 2006 | MODIS |  |  | 55±11 | Turner et al., 2006 |
| MTE | 120±6 |  | Jung et al., 2009 | BETHY |  |  | 60±3 | Tum et al., 2016 |
| BESS | 122±25 |  | Jiang and Ryu, 2016 | GIMMS |  |  | 33~49 | Smith et al., 2016 |

Table S3 Global P inputs and outputs estimated by ORCHIDEE-CNP and previous studies. The estimations by Lun et al. (2018) and Mekonnen et al. (2018) are based on the balance of P inputs and removal of P.

|  |  |  | ORCHIDEE-CNP |  | Lun et al., (2018) | Mekonnen and Hoekstra (2018) |
| --- | --- | --- | --- | --- | --- | --- |
| Study period |  |  | 2002-2010 |  | 2002-2010 | 2002-2010 |
| Study biomes |  |  | Both natural and managed biomes |  | Cropland and pasture | Only cropland |
| P inputs |  | P deposition input | 3.8 |  | 1.4 | - |
|  |  | Sludge P input | - |  | 1.4 | 1.2 |
|  |  | Chemical fertilizer P input | 20.1 |  | 16.8 | 17 |
|  |  | Manure P input | 6.7 |  | 19.8 | 5.9 |
| P outputs |  | P leaching | 3.8 |  | - | - |
|  |  | P leaching from natural land | 0.38 |  | - | - |
|  |  | P leaching from managed land | 3.3 |  | - | - |
|  |  | P erosion loss | - |  | - | - |
|  |  | P runoff and leaching loss | 3.8 |  | 5.4 | - |
|  |  | P erosion, leaching and runoff loss | - |  |  | 0.6 |
| P erosion-runoff-leaching to freshwater from diffuse sources |  |  |  |  | 12.5% | 2.8% |

Table S4 Averaged N and P load via drainage and runoff (Tg yr-1) over 2002-2010 simulated by ORCHIDEE-CNP and GOLUM-CNP.

|  | ORCHIDEE-CNP |  |  | GOLUM-CNP |
| --- | --- | --- | --- | --- |
|  | ALL | Natural biome | Managed biome |  |
| N leaching | 55 | 35 | 20 | 30 |
| P leaching | 3.5 | 0.38 | 3.2 | 0.39 |

Table S5 Inorganic sorption dynamic parameters for Oxisols, Molisols and other soils in ORCHIDEE-CNP.

|  | Oxisols | Molisols | Other soils | References |
| --- | --- | --- | --- | --- |
| Freundlich Isotherm coefficient (LH2O kg-1soil) | 348.9 | 185.5 | 72.3 | Kvacik et al. (in prep) |

Figure S1 Fluxes and pool sizes of C, N and P for 6 biomes by ORCHIDEE-CNP and GOLUM-CNP. The targeted biomes are tropical rainforests (TRF, a, g), temperate deciduous forests (TEDF, b, h), temperate coniferous forests (TECF, c, i), boreal coniferous forests (BOCF, d, j), tropical/C4 grasslands (TRG, e, k), temperate/C3 grasslands (TEG, f, l). Numbers in square brackets indicate the standard deviations for accounting the spatial spread of C, N and P fluxes.

Figure S2 Global pattern of aboveground forest biomass carbon density from ORCHIDEE-CNP and two data-driven map of GlobBiomass and GERCARBON.

Figure S3 Global pattern of soil organic carbon of 0~2 meters depth from GSDE and Soilgrids and derived from ORCHIDEE-CNP.

Figure S4 Comparisons between GPP and absorbed light by canopy (APAR; a), fraction of absorbed light by canopy (fAPAR; b) and ecosystem evapotranspiration (ET; c) used by 6 biomes: tropical rainforests (TRF), temperate deciduous forests (TEDF), temperate coniferous forests (TECF), boreal coniferous forests (BOCF), tropical/C4 grasslands (TRG) and temperate/C3 grasslands (TEG).  (d) shows the relations between GPP and NPP on biome scale. Reference data (same with Figure 14), ORCHIDEE-CNP and ORCHIDEE are discriminated by color black, red and blue. Error bars indicate the standard deviation of GPP, NPP and resources used by biome.

Figure S5 Global pattern of CO2 fertilization effect (ECO2), defined as the ratio of current GPP with atmospheric CO2 concentration of 396 ppm (GPP396) and GPP under pre-industrial atmospheric CO2 concentration of 296 ppm (GPP296), for natural C3 plants (a, c) and natural C4 plants (b, d) by ORCHIDEE-CNP (a, b) and ORCHIDEE (c, d).

Figure S6 Comparisons of global pattern for plant primary productivity between ORCHIDEE-CNP and data-driven products (MODIS-GPP, MTE-GPP, BESS-GPP, MODIS-NPP, BETHY-NPP, GIMMS-NPP). Black point, ‘’ and ‘∇’ indicates the GPP or NPP simulated by ORCHIDEE-CNP lie within the ranges / higher than the upper limits / lower than the lower limits of estimations by data-driven products, respectively. The ranges of data-driven GPP and NPP involves uncertainties within each product and among all products.

Figure S7 Sensitivity of GPP anomalies to anomalies of mean annual temperature (ST), annual precipitation (SP) and shortwave radiation (SR) plotted in a mean annual temperature (MAT) – mean annual precipitation (MAP) space for MTE-GPP, BESS-GPP, ORCHIDEE and ORCHIDEE-CNP.

Figure S8 Global patterns of sensitivities of GPP anomalies to anomalies of mean annual temperature (ST), annual precipitation (SP) and shortwave radiation (SR) derived from MTE-GPP, BESS-GPP, ORCHIDEE and ORCHIDEE-CNP.

Figure S9 Seasonal cycles of GPP by BESS-GPP, MTE-GPP, ORCHIDEE and ORCHIDEE-CNP for the northern hemisphere (30oN-90 oN; a), north tropical (0o-30oN; b), south tropical (0o-30 oS; c) and the southern hemisphere (30oS-90 oS; d).

(a) NH

(b) NT

(c) ST

LAI

(d) SH

Figure S10 Seasonal cycle of LAI derived from GIMMS, ORCHIDEE and ORCHIDEE-CNP for the northern hemisphere (30oN-90 oN; a), the north tropical (0o-30oN; b), the south tropical (0o-30 oS; c) and the southern hemisphere (30oS-90 oS; d).

SH

ST

NT

NH

SH

ST

NT

NH

gP kg DM-1

gN kg DM-1

Month

Month

(a) Leaf N concentration

(b) Leaf P concentration

Figure S11 Seasonal cycle of leaf N (a) and P concentration (b) simulated by ORCHIDEE-CNP with excluding managed lands (crop and pastures) for north hemisphere (NH; 30oN-90 oN), north tropical (NT; 0o-30oN), south tropical (ST; 0o-30 oS) and south hemisphere (SH; 30oS-90 oS).

Figure S12 Comparisons of net biome productivity (NBP) between ORCHIDEE-CNP, ORCHIDEE, GCP and inversion dataset for (a) globe (b) the northern hemisphere (30oN-90 oN), (c) tropical (30oS-30 oN), and (d) the southern hemisphere (30oS-90oS). Grey shading indicates ±1σ uncertainty of DGVM results for Trendy v6.

Figure S13 Changes of accumulated immobilized nutrient and net nutrient inputs (area-weighted average) for natural biomes during 1850-2016 on global, north hemisphere (NH, 30oN~90oN) and tropical region (30oS~30oN).

Figure S14 Accumulated immobilized nutrient, net nutrient inputs and deficit during 1850-2016 for N (a, b, c) and P (d, e, f). Grids dominated with managed biomes (cropland and pasture) (area fraction>50% in 2016) are masked.

Figure S15 Total N and P budgets for each biome. Error bars indicate the standard deviation over 2002-2010. TREB: tropical evergreen broadleaf forest; TRDB: tropical deciduous broadleaf forest; TEEN: temperate evergreen conifer forest; TEEB: temperate evergreen broadleaf forest; TEDB: temperate deciduous broadleaf forest; BEN: boreal evergreen conifer forest; BDN: boreal deciduous conifer forest; C3G: C3 grassland; C4G: C4 grassland; C3P: C3 pasture; C4P: C4 pasture; C3C: C3 cropland; C4C: C4 cropland.

Figure S16 Global BNF and tropical forest BNF simulated by ORCHIDEE-CNP, BNF model A by Peng et al. (2019), and revising BNF model B (Cleveland et al., 1999) and C (Wang and Houlton, 2009) based on plot-level measurements in tropical forests by Sullivan et al. (2014) (grey box).

Figure S17 Global pattern of N (Nload, a-c) and P loads (Pload, d-f) from land to rivers for basins with area larger than 50,000 km2. (c) and (f) show the comparisons of N and P leaching between ORCHIDEE-CNP and GlobalNEWS2 (Mayorga et al., 2010). Black line indicates 1:1 line. Pink dashed line indicates the linear regression line for basins with fertilized basin (N fertilization higher 1 gNm-2yr-1 or P fertilization higher than 0.5 g Pm-2yr-1), while grey dashed line indicates the linear regression line for basins with natural basin (N fertilization lower 1 gNm-2yr-1 or P fertilization lower than 0.5 g Pm-2yr-1). Black dashed lines indicate the linear regression line for all basins. R2 and RMSE refer to the coefficient of determination and root-mean-square error between estimations of ORCHIDEE-CNP and estimates from GlobalNEWS2 (Mayorga et al., 2010) for all basins.

Figure S18 Global pattern of N and P load rates from land to river (Nload and Pload) on catchment scale for ORCHIDEE-CNP and GlobalNEWS2 model for basins with area larger than 50,000 km2. (c) and (f) show the comparisons of Nload and Pload between ORCHIDEE-CNP and GlobalNEWS2. Black line indicates 1:1 line.

Figure S19 Violin plots of soil solution inorganic P turnover rates by soil order between ORCHIDEE-CNP (red) and measurements (black) (Helfenstein et al., 2018). The number of measurements and grid cells for each soil suborder are also shown. Open circles are medians of all grid cells within each biome, with balloons representing the probability density distribution of each value.

Figure S20 Global pattern of biochemical mineralization by ORCHIDEE-CNP (a) and normalized acid phosphatase activity (b).

Figure S21 Chemical and manure N and P input on catchment scale by GlobalNEWS2 and ORCHIDEE-CNP.