M I E V    D O C U M E N T A T I O N

------------------------------------

** NOTE **  The output variable SPIKE, having to do with the detection

of resonances, is still under research and will undoubtedly change in

the future.  Presently, SPIKE only detects the broadest spikes, of width

roughly 0.1 in size parameter;  ultimately narrower spikes should also

be detected, although the probability of hitting them is much smaller.

SPIKE is mainly of use in avoiding spikes during numerical integration

over a size distribution.

Author:  Dr. Warren J. Wiscombe (wiscombe@climate.gsfc.nasa.gov)

NASA Goddard Space Flight Center

Code 913

Greenbelt, MD 20771

FTP availability:  The entire package is available by anonymous ftp

from Internet site climate.gsfc.nasa.gov in subdirectory

pub/wiscombe.  (ftp to 'climate', login as 'anonymous', give

your e-mail address as password, then 'cd' to pub/wiscombe.)

The MIEV package contains the following files (besides the present one):

(1) MIEV0.f, the main subroutine which a user calls, plus ancillary

subroutines

(1a) MIEV0noP.f:  MIEV0.f with all the code relating to Legendre

moments PMOM removed (smaller and requires less array storage);

just created 12/89 and seems to be working fine but has not had

the benefit of years of user testing like MIEVO.f;  argument

list same as MIEV0.f in order to allow swapping of this with

MIEV0.f without changing calling program(s)

(2) ErrMsg.f:  a set of 4 error-handling routines needed by both

MIEV0.f and MIEV0noP.f

(3) MVTstOld.f, the main program for running the 8 test cases at

the end of Reference (1) below

(4) MVTstOld.out, the output generated by MVTstOld.f

(5) MVTstNew.f, the main program for running an exhaustive set of

19 test cases.

(6) MVTstNew.out, the output generated by MVTstNew.f

(7) PMOMTest.f, a program to test the Legendre coefficients computed

by  *MIEV0*  against those computed approximately by numerical

quadrature of the phase matrix

Note that MIEV1, the Cray-customized version of MIEV0 described in

Ref.(1) below, is omitted from this package.  Very few users chose to

use MIEV1 because its speed advantage over MIEV0 was only

about 20% in typical cases.  The speed advantage could grow to several

hundred percent if only cross-sections and asymmetry factor were needed,

since MIEV1 took advantage of the CRAY's ability to vectorize summing

loops.

All modules have some internal documentation in addition to what

you will find in this file.  Also, all the declaration statements

were standardized using the NAG Fortran Tools.

MIEV0 computes the following quantities involved in eletromagnetic

scattering from a homogeneous sphere:

* scattering and extinction efficiencies;

* asymmetry factor;

* forward- and backscatter amplitude;

* scattering amplitudes vs. scattering angle for incident

polarization parallel and perpendicular to the plane

of scattering;

* coefficients in the Legendre polynomial expansions of

either the unpolarized phase function or the polarized

phase matrix;

* some quantities needed in polarized radiative transfer;

* information about whether or not a resonance has been hit.

NOTE -- MIEV0 differs from the original code published

in Ref. (1) below in the following ways :

* computes Legendre moments, based on vast

improvements to the formulas of Sekera (see Refs. 3-5)

and correction of errors in the formulas of Ref. 3

* returns a measure of how nasty of a spike (resonance) you

are sitting on ( this is invaluable when integrating over

size and you want to exclude unrepresentative points);

this part of the program is a work-in-progress and

is far from finished, but it may prove useful even

in its present form

* allows real refractive indices less than unity

* adds a totally reflecting special case

* performs a self-test on the first call to the routine

* adds several new input and output variables, and makes

all I/O through arguments of the subroutine

* allows complete freedom in specifying angles

* allows printing of all output variables at user option

* some variables names are more mnemonic

Also, major improvements have been made, based on

modern ideas of documentation and program structure (e.g.,

Kernighan and Plauger, The Elements of Programming Style).

Those interested in my thoughts in this area may find

PostScript documents in the anonymous ftp directory cited

above, under pub/wiscombe/Writing_Programs.

REFERENCES

----------

(1) Wiscombe, W., 1979: Mie Scattering Calculations--Advances

in Technique And Fast, Vector-Speed Computer Codes,

Ncar Tech Note Tn-140+Str, National Center For

Atmospheric Research, Boulder, Colorado (NCAR no

longer distributes this, so contact the author or

NTIS for a copy)

(2) Wiscombe, W., 1980: Improved Mie Scattering Algorithms,

Appl. Opt. 19, 1505-1509

(3) Dave, J.V., 1970a:  Coefficients of the Legendre and

Fourier Series for the Scattering Functions of

Spherical Particles, Appl. Opt. 9, 1888-1896

(4) Dave, J.V., 1970b:  Intensity and Polarization of the

Radiation Emerging from a Plane-Parallel Atmosphere

Containing Monodisperse Aerosols, Appl. Opt. 9, 2673-84

(5) Van De Hulst, 1957, 1982:  Light Scattering by Small

Particles, Dover Press, New York.

(6) Bohren, C. and D. Huffman, Absorption and Scattering of

Light by Small Particles, Wiley, New York.  (has a

Mie program in the back of the book)

I N P U T   V A R I A B L E S

-----------------------------

( Even if an input variable is not needed for a particular

application, make sure it has a legitimate value that can

be written out and read in -- no indefinites, etc. )

XX        Mie size parameter ( 2 * pi * radius / wavelength )

CREFIN    Complex refractive index ( imag part can be + or -,

but internally a negative imaginary index is assumed ).

If imag part is - ,  scattering amplitudes as in Van

de Hulst are returned;  if imag part is + , complex

conjugates of those scattering amplitudes are returned

(the latter is the convention in physics).

** NOTE ** In the 'PERFECT' case, scattering amplitudes

in the Van de Hulst (Ref. 6 above) convention will

automatically be returned unless  Im(CREFIN)  is

positive;  otherwise, CREFIN plays no role.

PERFCT    TRUE, assume refractive index is infinite and use

special case formulas for Mie coefficients  'a'

and  'b'  ( see Kerker, M., The Scattering of

Light and Other Electromagnetic Radiation, p. 90 ).

This is sometimes called the 'totally reflecting',

sometimes the 'perfectly conducting' case.

( see CREFIN for additional information )

MIMCUT    (positive) value below which imaginary refractive

index is regarded as zero ( computation proceeds

faster for zero imaginary index )

ANYANG    TRUE, any angles whatsoever may be input through

XMU.  FALSE, the angles are monotone increasing

and mirror symmetric about 90 degrees (this option

is advantageous because the scattering amplitudes

S1,S2 for the angles between 90 and 180 degrees

are evaluable from symmetry relations, and hence

are obtained with little added computational cost.)

NUMANG    No. of angles at which scattering amplitudes

S1,S2 are to be evaluated  ( set = 0 to skip

calculation of S1,S2 ).  Make sure NUMANG does

not exceed the parameter MAXANG in the program.

XMU(N)    Cosines of angles ( N = 1 TO NUMANG ) at which S1,S2

are to be evaluated.  If ANYANG = FALSE, then

(a) the angles must be monotone increasing and

mirror symmetric about 90 degrees (if 90-A is

an angle, then 90+A must be also)

(b) if NUMANG is odd, 90 degrees must be among

the angles

NMOM       Highest Legendre moment PMOM to calculate,

numbering from zero ( NMOM = 0 prevents

calculation of PMOM )

IPOLZN     POSITIVE, Compute Legendre moments PMOM for the

Mueller matrix elements determined by the

digits of IPOLZN, with 1 referring to M1,

2 to M2, 3 to S21, and 4 to D21 (Ref. 3).

E.g., if IPOLZN = 14 then only moments for

M1 and D21 will be returned.

0,        Compute Legendre moments PMOM for the

unpolarized unnormalized phase function.

NEGATIVE, Compute Legendre moments PMOM for the

Sekera phase quantities determined by the

digits of ABS(IPOLZN), with 1 referring to

R1, 2 to R2, 3 to R3, and 4 to R4 (REF. 4).

E.g., if IPOLZN = -14 then only moments for

R1 and R4 will be returned.

( NOT USED IF  NMOM = 0 )

MOMDIM     Determines first dimension of PMOM, which is dimensioned

internally as PMOM( 0:MOMDIM, * ) (second dimension must

be the larger of unity and the highest digit in

IPOLZN; if not, serious errors will occur).

Should be set even when  NMOM = 0.  A zero value is OK.

PRT(L)     Print flags (LOGICAL).  L = 1  prints  S1,S2, their

squared absolute values, and degree of polarization,

provided NUMANG is non-zero.   L = 2  prints all

output variables other than  S1,S2.

O U T P U T   V A R I A B L E S

-------------------------------

QEXT      (REAL) extinction efficiency factor  ( Ref. 2, Eq. 1A )

QSCA      (REAL) scattering efficiency factor  ( Ref. 2, Eq. 1B )

GQSC      (REAL) asymmetry factor times scattering efficiency

( Ref. 2, Eq. 1C )  ( allows calculation of radiation

pressure efficiency factor  QPR = QEXT - GQSC )

=====================================================================

==== NOTE --  S1, S2, SFORW, SBACK, TFORW, AND TBACK are calculated

====          internally for negative imaginary refractive index;

====          for positive imaginary index, their complex conjugates

====          are taken before they are returned, to correspond to

====          customary usage in some parts of physics ( in parti-

====          cular, in papers on cam approximations to Mie theory ).

=====================================================================

S1(N),    (COMPLEX) Mie scattering amplitudes at angles specified

S2(N)     by XMU(N) ( N=1 to NUMANG )  ( Ref. 2, Eqs. 1d-e ).

SFORW     (COMPLEX) forward-scattering amplitude S1 at

0 degrees.  ( S2(0 deg) = S1(0 deg) )

SBACK     (COMPLEX) backscattering amplitude S1 at

180 degrees.   ( S2(180 deg) = - S1(180 deg) )

TFORW(I)  (COMPLEX) values of

I=1:  T1 = ( S2 - (MU)*S1 ) / ( 1 - MU**2 )

I=2:  T2 = ( S1 - (MU)*S2 ) / ( 1 - MU**2 )

At angle theta = 0 ( MU = COS(theta) = 1 ), where the

expressions on the right-hand side are indeterminate.

( these quantities are required for doing polarized

radiative transfer (Ref. 4, Appendix ). )

TBACK(I)  (COMPLEX) values of  T1 (for I=1) or  T2 (for I=2) at

angle  theta = 180 degrees ( MU = COS(theta) = - 1 ).

SPIKE     (REAL) magnitude of the smallest denominator of

either Mie coefficient (a-sub-n or b-sub-n),

taken over all terms in the Mie series past

N = size parameter XX.  Values of SPIKE below

about 0.3 signify a ripple spike, since these

spikes are produced by abnormally small denominators

in the Mie coefficients (normal denominators are of

order unity or higher).  Defaults to 1.0 when not

on a spike.  Does not identify all resonances

(we are still working on that).

PMOM(M,NP) (REAL) moments  M = 0 to NMOM  of unnormalized NP-th

phase quantity PQ  ( moments with  M .GT. 2*NTRM  are

zero, where  NTRM = no. terms in Mie series =

XX + 4*XX**1/3 + 1 ) :

PQ( MU, NP ) = sum( M=0 to infinity ) ( (2M+1)

* PMOM( M,NP ) * P-sub-M( MU ) )

WHERE  MU = COS( scattering angle )

P-sub-M = M-th Legendre polynomial

and the definition of 'PQ' is as follows:

IPOLZN.GT.0:  PQ(MU,1) = CABS( S1(MU) )**2

PQ(MU,2) = CABS( S2(MU) )**2

PQ(MU,3) = RE( S1(MU)*CONJG( S2(MU) ) )

PQ(MU,4) = - IM( S1(MU)*CONJG( S2(MU) ) )

( called M1, M2, S21, D21 in literature )

IPOLZN=0:  PQ(MU,1) = ( CABS(S1)**2 + CABS(S2)**2 ) / 2

( the unnormalized phase function )

IPOLZN.LT.0:  PQ(MU,1) = CABS( T1(MU) )**2

PQ(MU,2) = CABS( T2(MU) )**2

PQ(MU,3) = RE( T1(MU)*CONJG( T2(MU) ) )

PQ(MU,4) = - IM( T1(MU)*CONJG( T2(MU) ) )

( called R1, R2, R3, R4 in literature )

The sign of the 4th phase quantity is a source of

confusion.  It flips if the complex conjugates of

S1,S2  or  T1,T2  are used, as occurs when a

refractive index with positive imaginary part is

used (see discussion below).  The definition above

is consistent with a negative imaginary part.

See Ref. 5 for correct formulae for PMOM ( Eqs. 2-5

of Ref. 3 contain typographical errors ).  Ref. 5 also

contains numerous improvements to the Ref. 3 formulas.

NOTE THAT OUR DEFINITION OF MOMENTS DIFFERS FROM REF. 3

in that we divide out the factor (2M+1) and number the

moments from zero instead of one.

** WARNING **  Make sure the second dimension of PMOM

in the calling program is at least as large as the

absolute value of IPOLZN.

For small enough values of XX, or large enough values

of M,  PMOM  will tend to underflow.  Thus, it is

unwise to assume the values returned are non-zero and,

for example, to divide some quantity by them.

INTEGRATING OVER SIZES

----------------------

The normalized phase function for a single size parameter is

P(one size) = 4 / ( XX**2 * QSCA ) * ( i1 + i2 ) / 2

where  i1 + i2 = CABS(S1)**2 + CABS(S2)**2.  But it is

( i1 + i2 ), not  P(one size), that must be integrated

over sizes when a size distribution is involved.

(Physically, this means that intensities are added,

not probabilities. )  An a posteriori normalization

then gives the correct size-averaged phase function.

Similarly, it is the CROSS-SECTIONS, proportional to

XX**2  times QEXT,QSCA,QPR, which should be integrated

over sizes, not QEXT,QSCA,QPR themselves.

Similar remarks apply to PMOM.   The normalized moments are

4 / ( XX**2 * QSCA ) * PMOM,  but it is PMOM itself, not

these normalized moments, which should be integrated over

a size distribution.

Unless avoided, ripple spikes can cause a systematic upward

bias in any integration over size parameter, because these

spikes tend to be smeared out by typical quadrature rules and

thus over-represented in the final result.  Checking the output

parameter SPIKE allows the user to filter out these cases.

NOTES ON PROGRAM USE

--------------------

*** PMOM dimensioning:

One user dimensioned PMOM(1,1) in his calling program and

managed to clobber SFORW because he set MOMDIM=1 and

internally PMOM is dimensioned PMOM( 0:MOMDIM, * ).

Fortran seems to allow PMOM(0:0,*) so he could have

saved himself by setting MOMDIM=0, but this is confusing

and it is better to start your PMOM array at 0 just as

the program does internally.

Be sure to use the test problem drivers as templates

when designing your calls to MIEV0 in order to avoid this

kind of problem.

*** ON PORTABILITY :

This package is written entirely in ANSI standard FORTRAN 77

and should work on any computer.

*** ON PRECISION :

"You should be aware that a complex program can produce

different results on one computer than another because of

differences in internal precision.  The difference can be

minimized, but not necessarily eliminated, by using double-

precision arithmetic and by using numerical methods that tend

to retain maximum precision."

(IBM Professional FORTRAN Manual)

This package was developed on computers offering 14-digit

single-precision computation.   On IBM-type machines with their

7-digit single precision, parts of the computation ( like the

upward recurrence for the Ricatti-Bessel functions ) might need

to be done in double precision, depending on how big the Mie

size paramter  XX  is.   See Ref. (1) for further discussion

of this point.

The package has only been tested for XX up to 20,000 and

for real and imaginary part of CREFIN up to 10  ( this

accomodates almost all imaginable applications ).  Slow

deterioration of accuracy may be expected if the program

is pushed beyond these limits.  ( Accuracy may degrade well

before  XX = 20000  with IBM-type 7-digit precision. )

Precision problems are most likely to OVERTLY afflict

users

(a) in the self-test subroutine TESTMI, where it may be

necessary to lower the required agreement with tabulated

results by changing the variable  ACCUR.  For example,

to run on the IBM PC in single precision using 'IBM

Professional FORTRAN', a value  ACCUR = 1.E-4  was

necessary.

(b) in the testing routines MVTst..., where the user's

precision may be unsatisfactory for numerically

'sensitive' quantities.

The quantities most sensitive to precision are those involving

series of positive and negative terms with much cancellation.

The smaller the end result of summing compared to the average

term size, the worse the problem.  The problem can occur

in either of the scattering amplitudes (S1,S2) away from the

forward scattering angle, esp. near a relative minimum.  When

the real and imaginary parts of  S1  or  S2  differ by

orders of magnitude, the smaller part is likely to be less

accurate than the larger.

The least accurate output quantities will be :

**  TFORW and TBACK, because the numerical factors

involved are on the order of  XX**3

**  PMOM( M, 4 )  for any  M  and larger XX

**  PMOM( M, NP )  for  M  approaching  2*XX

The most accurate will be  QEXT, QSCA, GQSC, being sums

of all positive terms.

Please do not call the author about precision problems.

They are endemic and cannot be solved as long as different

computers do arithmetic differently.

*** ON MEMORY REQUIREMENTS :

The parameter MaxTrm in  MIEV0, LPCOEF  must be

set to  10,100  in order to do the test problems with

size parameter = 10,000.  Memory used by these routines can

be significantly, often dramatically, reduced by lowering

MaxTrm  to a value no bigger than  XMAX + 4*XMAX**1/3,

where  XMAX  is the largest size parameter expected.

If PMOM is never needed, the version MIEV0noP.f should

be used instead of MIEV0.f.  This can substantially cut memory

requirements.

*** The self-test on the first call to the program is a novel

feature intended to catch bugs which users may introduce into

the code.  But it does not begin to test all the possible

branches in the code.  The test programs included with

this package should be used for thorough checkout of all

branches.

*** The arithmetic statement function  F3  is built into

*MIEV0*, but not used.  It corresponds to the function

f-sub-3  in Ref. 2, Eq. 8, and should be used instead

of  F2  when only intensity and degree of polarization

are required.   This can be implemented just by

changing  F2 to F3 in a single executable statement.

*** To avoid littering up the code with temporary variables,

a reasonably optimizing compiler (one that recognizes

invariant and repeated sub-expressions in DO-loops)

has been assumed.  This may make the code look wasteful

to those accustomed to dumb FORTRAN compilers.

*** Equivalenced arrays have been used in one place (module

LPCOEF).  (EQUIVALENCE is a dangerous feature of FORTRAN

and should generally be avoided.)

*** MIEV0 sacrifices some computational speed on vector computers

in order to use the minimum possible amount of computer

memory;  however, it still allows loops over scattering

angle to vectorize;  and on vector computers which vectorize

summing loops ( like the Cray ), the potentially

time-consuming inner loops in the Legendre coefficient

subroutine will also vectorize  ( these two kinds of loops

account for the lion's share of computing time in a typical

application ).