Table S1 The settings of combustion completeness in different land cover types.

| fracforest | >60% | 40%-60% | <40% |
| --- | --- | --- | --- |
| cropland | 0.93 | 0 | 0 |
| grassland | 0.9 | e−0.13×FB | 0.98 |
| forest | 0.3 | 0.05 | 0 |

Note: fracforest indicates the fraction of percentage of forest cover. FB stands for the fraction of burned within an individual grid cell.

Table S2 The emission factors of wildfire-induced emissions for various GHG types (unit: g kg−1 dry matter).

| GHG | Grassland | Trop. F | Temp. F | Bor. F | Cropland | Wheat | Rice | Maize | Sugarcane |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO2 | 1666.3±81.2 | 1632.1±65.0 | 1583.7±123.3 | 1519.2±136.5 | 1452.8±222.7 | 1454.4±50.0 | 1474.0±221 | 1336.0±36.6 | 1270.0±170.0 |
| CH4 | 2.63±2.12 | 6.10±1.82 | 5.05±2.79 | 5.59±2.65 | 5.72±5.60 | 2.46±1.19 | 3.51±3.12 | 3.72±1.72 | 0.40±0.20 |
| N2O | 0.17±0.09 | 0.20±0.1 | 0.19±0.18 | 0.27±0.12 | 0.09±0.04 | 0.07±0.02 | 0.07±0.02 | 0.14±0.03 | 0.09±0.04 |

Note: Grassland represents all short vegetation dominated areas including grassland, shrubland and other herbaceous areas. Trop. F, Temp. F and Bor. F are the abbreviations for tropical, temperate and boreal forest. The different types of crop residues including wheat, rice, maize and sugarcane were contained within the extent of cropland pixels, using the ratio between planting area and grid area for calculation. The references are listed at the end of the text [1–14].

Table S3 Average burned area and wildfire-induced GHG emissions at regional and provincial level from 2012 to 2022.

| Region/Province | Burned area (Mha yr-1) | CO2 (Tg yr-1) | CH4 (Gg yr-1) | N2O (Gg yr-1) | Major fuel typeⱡ | Peak month# |
| --- | --- | --- | --- | --- | --- | --- |
| Northeast China | 3.29 ± 1.49 | 43.07 ± 19.22 | 159.34 ± 73.01 | 3.33 ± 1.31 | Crop | April |
| Nei Mongol | 0.48 ± 0.16 | 4.98 ± 2.16 | 16.48 ± 7.53 | 0.54 ± 0.30 | Crop | March |
| Liaoning | 0.09 ± 0.04 | 1.28 ± 0.57 | 4.85 ± 2.22 | 0.10 ± 0.04 | Crop | April |
| Jilin | 0.65 ± 0.50 | 9.10 ± 6.80 | 34.28 ± 25.09 | 0.58 ± 0.41 | Crop | April |
| Heilongjiang | 2.06 ± 1.18 | 27.71 ± 15.22 | 103.73 ± 57.72 | 2.12 ± 1.03 | Crop | April |
| East China | 0.73 ± 0.95 | 10.51 ± 10.75 | 38.20 ± 38.60 | 0.73 ± 0.69 | Crop | June |
| Shanghai | 0.00 ± 0.00 | 0.01 ± 0.01 | 0.03 ± 0.03 | 0.00 ± 0.00 | Crop | November |
| Jiangsu | 0.13 ± 0.21 | 1.43 ± 2.12 | 5.22 ± 7.64 | 0.09 ± 0.13 | Crop | June |
| Zhejiang | 0.01 ± 0.01 | 0.40 ± 0.21 | 1.44 ± 0.73 | 0.03 ± 0.02 | Crop | March |
| Anhui | 0.41 ± 0.67 | 4.26 ± 6.75 | 15.29 ± 24.10 | 0.26 ± 0.41 | Crop | June |
| Jiangxi | 0.04 ± 0.03 | 1.32 ± 0.98 | 4.76 ± 3.52 | 0.11 ± 0.08 | Crop | January |
| Hubei | 0.08 ± 0.05 | 1.27 ± 0.96 | 4.72 ± 3.53 | 0.09 ± 0.07 | Crop | February |
| Hunan | 0.06 ± 0.03 | 1.82 ± 1.20 | 6.74 ± 4.35 | 0.14 ± 0.10 | Crop | January |
| North China | 0.74 ± 0.58 | 6.14 ± 4.64 | 21.57 ± 16.34 | 0.41 ± 0.29 | Crop | June |
| Beijing | 0.00 ± 0.00 | 0.03 ± 0.03 | 0.13 ± 0.11 | 0.00 ± 0.00 | Crop | October |
| Tianjin | 0.01 ± 0.01 | 0.11 ± 0.06 | 0.41 ± 0.22 | 0.01 ± 0.00 | Crop | January |
| Hebei | 0.11 ± 0.07 | 0.83 ± 0.46 | 2.83 ± 1.58 | 0.06 ± 0.03 | Crop | June |
| Shanxi | 0.08 ± 0.03 | 0.70 ± 0.33 | 2.29 ± 1.05 | 0.06 ± 0.03 | Crop | March |
| Shandong | 0.17 ± 0.03 | 1.54 ± 1.14 | 5.57 ± 4.08 | 0.10 ± 0.07 | Crop | June |
| Henan | 0.37 ± 0.43 | 2.93 ± 3.26 | 10.35 ± 11.39 | 0.18 ± 0.20 | Crop | May |
| South China | 0.16 ± 0.05 | 5.93 ± 1.97 | 20.49 ± 6.83 | 0.53 ± 0.17 | Crop | January |
| Fujian | 0.02 ± 0.01 | 0.66 ± 0.21 | 2.24 ± 0.74 | 0.06 ± 0.02 | Forest | January |
| Guangdong | 0.05 ± 0.03 | 1.92 ± 1.19 | 6.72 ± 4.15 | 0.16 ± 0.10 | Crop | January |
| Guangxi | 0.08 ± 0.01 | 3.02 ± 0.63 | 10.32 ± 2.17 | 0.27 ± 0.06 | Crop | January |
| Hainan | 0.01 ± 0.01 | 0.25 ± 0.14 | 0.91 ± 0.51 | 0.02 ± 0.01 | Crop | February |
| Taiwan | 0.00 ± 0.00 | 0.07 ± 0.02 | 0.27 ± 0.09 | 0.00 ± 0.00 | Crop | March |
| Hong Kong | 0.00 ± 0.00 | 0.01 ± 0.01 | 0.03 ± 0.04 | 0.00 ± 0.00 | Forest | March |
| Macau | 0.00 ± 0.00 | 0.00 ± 0.00 | 0.00 ± 0.00 | 0.00 ± 0.00 | Grass | January |
| Northwest China | 0.12 ± 0.06 | 1.36 ± 0.77 | 4.84 ± 2.76 | 0.10 ± 0.06 | Crop | April |
| Shaanxi | 0.03 ± 0.02 | 0.29 ± 0.18 | 1.02 ± 0.64 | 0.03 ± 0.02 | Crop | March |
| Gansu | 0.02 ± 0.01 | 0.14 ± 0.04 | 0.48 ± 0.13 | 0.01 ± 0.00 | Crop | March |
| Qinghai | 0.00 ± 0.00 | 0.03 ± 0.01 | 0.08 ± 0.03 | 0.00 ± 0.00 | Crop | October |
| Ningxia | 0.01 ± 0.01 | 0.09 ± 0.06 | 0.29 ± 0.20 | 0.01 ± 0.00 | Crop | October |
| Xinjiang | 0.06 ± 0.04 | 0.82 ± 0.58 | 2.97 ± 2.05 | 0.06 ± 0.04 | Crop | April |
| Southwest China | 0.26 ± 0.09 | 11.11 ± 4.45 | 35.02 ± 13.47 | 1.17 ± 0.50 | Forest | February |
| Chongqing | 0.00 ± 0.00 | 0.19 ± 0.10 | 0.72 ± 0.36 | 0.01 ± 0.01 | Crop | August |
| Sichuan | 0.05 ± 0.03 | 2.11 ± 1.23 | 6.80 ± 3.88 | 0.23 ± 0.16 | Forest | January |
| Guizhou | 0.04 ± 0.02 | 1.18 ± 0.58 | 4.17 ± 2.10 | 0.10 ± 0.05 | Crop | March |
| Yunnan | 0.16 ± 0.07 | 7.57 ± 3.71 | 23.13 ± 11.08 | 0.81 ± 0.41 | Forest | February |
| Xizang | 0.00 ± 0.00 | 0.06 ± 0.05 | 0.19 ± 0.15 | 0.01 ± 0.01 | Forest | March |

ⱡ The land cover types (Cropland, Grassland, Forests) with the most wildfire-induced CO2 emission.

# The single month with the most wildfire-induced CO2 emission.

Fig. S1 Samples of processing active fire and burned area products.

In subplot (a), the red cross indicates the longitude and latitude information derived from active fire records. The outer boundaries represent the extent of a 375-meter resolution VIIRS pixel, while the dark gray areas illustrate the calculated active fire pixel area. Subplot (b) provides an example of a 1-kilometer resolution pixel that integrates burned area and active fire products. The light gray area represents the internal conditions of the 500-meter burned area product, while the dark gray area corresponds to the active fire pixel described in subplot (a). Note that overlapping areas are not double counted.

Fig. S2. Samples of the removal of industrial hotspots

The background images are composites of Sentinel-2 imagery taken over Xuzhou, Jiangsu Province. Subplot (b) presents the initial results after combining the burned area and active fire datasets. Subplot (c) reveals that some burned areas are false detections, as identified using Liu's industrial hotspot dataset [15] (including ferrous metal-related plants and nonmetal mineral production sites in this sample), and should be filtered out.

Fig. S3 The national trends of burned area and three wildfire-induced GHGs.

Subplots b) to e) demonstrates the relative proportions of different types of fuels contributing the overall trends.

Fig. S4 The classification of six major regions discussed.

All the provincial administrative units are marked in lower font within the regional scope.

Fig. S5 Seasonal cycle of national and regional wildfire-induced CH4 emissions.

Note that the lines and bars share the same meaning with that in Fig. 2. Emissions from six regions are depicted on distinct Y-axes to accurately capture the seasonal variations in emissions patterns. Four sets of colors indicate four seasons.

Fig. S6 Seasonal cycle of national and regional wildfire-induced N2O emissions.

Note that the lines and bars share the same meaning with that in Fig. 2. Emissions from six regions are depicted on distinct Y-axes to accurately capture the seasonal variations in emissions patterns. Four sets of colors indicate four seasons.

References

1. Andreae MO. Emission of trace gases and aerosols from biomass burning – an updated assessment. Atmospheric Chemistry and Physics 2019;19:8523–46.

2. Akagi SK, Yokelson RJ, Wiedinmyer C et al. Emission factors for open and domestic biomass burning for use in atmospheric models. Atmos Chem Phys 2011;11:4039–72.

3. Akagi SK, Burling IR, Mendoza A et al. Field measurements of trace gases emitted by prescribed fires in southeastern US pine forests using an open-path FTIR system. Atmos Chem Phys 2014;14:199–215.

4. Wilson D, Dixon SD, Artz RRE et al. Derivation of greenhouse gas emission factors for peatlands managed for extraction in the Republic of Ireland and the United Kingdom. Biogeosciences 2015;12:5291–308.

5. Stockwell CE, Christian TJ, Goetz JD et al. Nepal Ambient Monitoring and Source Testing Experiment (NAMaSTE): emissionsof trace gases and light-absorbing carbon from wood and dung cooking fires,garbage and crop residue burning, brick kilns, and other sources. Atmos Chem Phys 2016;16:11043–81.

6. Hamada Y, Darung U, Limin SH et al. Characteristics of fire-generated gas emission observed during a large peatland fire in 2009 at Kalimantan, Indonesia. Atmospheric Environment 2013;74:177–81.

7. Li X, Wang S, Duan L et al. Particulate and Trace Gas Emissions from Open Burning of Wheat Straw and Corn Stover in China. Environ Sci Technol 2007;41:6052–8.

8. Zhang Y, Shao M, Lin Y et al. Emission inventory of carbonaceous pollutants from biomass burning in the Pearl River Delta Region, China. Atmospheric Environment 2013;76:189–99.

9. Li J, Bo Y, Xie S. Estimating emissions from crop residue open burning in China based on statistics and MODIS fire products. Journal of Environmental Sciences 2016;44:158–70.

10. Li J, Li Y, Bo Y et al. High-resolution historical emission inventories of crop residue burning in fields in China for the period 1990–2013. Atmospheric Environment 2016;138:152–61.

11. Das B, Bhave PV, Puppala SP et al. A model-ready emission inventory for crop residue open burning in the context of Nepal. Environmental Pollution 2020;266:115069.

12. Streets DG, Gupta S, Waldhoff ST et al. Black carbon emissions in China. Atmospheric Environment 2001;35:4281–96.

13. Dennis A, Fraser M, Anderson S et al. Air pollutant emissions associated with forest, grassland, and agricultural burning in Texas. Atmospheric Environment 2002;36:3779–92.

14. Yokelson RJ, Burling IR, Gilman JB et al. Coupling field and laboratory measurements to estimate the emission factors of identified and unidentified trace gases for prescribed fires. Atmospheric Chemistry and Physics 2013;13:89–116.

15. Liu Y, Hu C, Zhan W et al. Identifying industrial heat sources using time-series of the VIIRS Nightfire product with an object-oriented approach. Remote Sensing of Environment 2018;204:347–65.