Supplement S2 of

Regional CO2 and CH4 inversion system using WRF-Chem (v4.4)/ DART (v9.8.0) and continuous high-precision observations over the Korean Peninsula

Doyoon Kwon et al.

Correspondence to: Jinkyu Hong (jhong@yonsei.ac.kr)

• This supplement evaluates the joint CO2/CH4 inversion with controlled twin experiments and real-case filter-configuration diagnostics (Section S1–S3; Table S2; Figs S15–S24).

S1 Controlled-twin evaluation of the inversion under prescribed error sources

The performance of an emission inversion in the real atmosphere is shaped by many sources of error. These include transport-model error, systematic boundary-condition bias, and observation representativeness error, together with the spatial and temporal structure of the prior fluxes, the model resolution, and the choice of filter parameters (Michalak et al., 2017). To establish how the WRF-Chem/DART inversion responds to such errors under controlled conditions, a set of observing-system simulation experiments (OSSEs; Hoffman and Atlas, 2016) is conducted here, in which a known emission distribution (the truth) is retrieved from synthetic observations and a single error source is introduced at a time against a known-truth, error-free baseline (an identical-twin configuration in which the nature run and the assimilation use the same model). The target is a prescribed, synthetic emission field that the experiments aim to recover; it is not an estimate of the real-world emissions. This design isolates the response of the state-augmentation scheme to one prescribed error of known magnitude, in keeping with the diagnostic use of OSSEs to quantify the uncertainty that a specific error contributes within an inversion framework (Michalak et al., 2017; Bisht et al., 2023; Vardag and Maiwald, 2024). Ensemble-based joint estimation of concentrations and surface fluxes of this kind underlies operational carbon data-assimilation systems (van der Velde et al., 2018) and regional CO2 and CH4 inversions, including over East Asia (Liu et al., 2019; Chen et al., 2023; Liang et al., 2023).

These controlled experiments partly evaluate how the inversion responds to the error sources it faces in its real-data application (Section 6): each isolates the system's response to one such error at a known magnitude, a separation that the real-data inversion, where the errors act together, cannot provide. They characterize the behavior to be expected in the real application and inform the interpretation of the Section 6 results.

S1.1 Construction of known surface emission (truth) and recovery metric

The nature run that defines the target is generated with WRF-Chem using anthropogenic emissions scaled to 70 % of the EDGAR inventory, so that the prior, initialized from the unscaled inventory, over-estimates the target by a uniform 43 % (the inverse of the 70 % scaling) in both species. The prior ensemble is formed by perturbing the emission field with a 30 % standard deviation, set commensurate with the imposed reduction so that the target lies about one standard deviation below the prior mean and within the prior ensemble, and is therefore recoverable; a substantially tighter prior would place it in the ensemble tail and preclude recovery. The CO2 and CH4 emission fields are appended to the model state and updated jointly with the concentrations (state augmentation), as in ensemble state-and-parameter inversions (Chen et al., 2023). Synthetic CO2 and CH4 surface mole fractions are sampled from the nature run at the same three operational WMO/GAW stations as the real-data inversion (Anmyeondo, Gosan, and Ulleungdo) and assimilated, so the observing network is identical to that of the application. Seven experiments are run under otherwise identical filter settings (Table S2). The assimilation is cycled every 6 h from 1 July 2020, with the first two days discarded as filter spin-up, so that the analysis covers 3–17 July 2020, the 60 six-hourly cycles common to all experiments. The baseline (CTL) is known-truth and error-free in its components: the assimilation shares the nature run's transport, physics, and lateral-boundary product, with no structural model error. The initial and lateral-boundary concentrations (5 %, 1σ) and the emissions are still perturbed across the ensemble (Section 4) to supply the prior spread the filter requires to update from the observations. Each remaining experiment departs from CTL through a single prescribed error. The transport experiment (TR) substitutes a different planetary-boundary-layer scheme in the assimilation (Shin–Hong in the nature run, MYNN2.5 in the assimilation), transport error being a leading structural uncertainty in regional inversions, with the boundary-layer scheme among its dominant contributors (Locatelli et al., 2013; Deng et al., 2017; Díaz-Isaac et al., 2018). The two schemes produce appreciably different transport: averaged over the window, their planetary-boundary-layer depth differs by about 180 m and their 10 m wind by about 1.4 m s⁻¹, displacing the simulated surface mixing ratios by about 0.8 ppm for CO2 and 5 ppb for CH4 (locally up to about 3 ppm and 20 ppb). The boundary-condition experiment (BC) imposes a background bias of known size on the inflowing air, −2 ppm for CO2 and −30 ppb for CH4. Both magnitudes are consistent with documented regional lateral-boundary biases: the CO2 value with CAMS biases of about 2 ppm over the Korean Peninsula (Tang et al., 2018; Park et al., 2020; Callewaert et al., 2026) and in the global CAMS-EGG4 reanalysis (Custódio et al., 2022), and the CH4 value with the CAMS methane evaluation of Segato et al. (2025). Each is comparable to the modest enhancement the surface network senses above the regional background (1–4 ppm for CO2 and about 19 ppb for CH4), so a bias of this size aliases at leading order into the recovered emissions. The observation-error experiment (OE) inflates the assimilated observation-error standard deviation for both species by a common factor of 2 (from 5 to 10 ppm for CO2 and from 30 to 60 ppb for CH4), representing larger representativeness uncertainty (Miller et al., 2015; Agustí-Panareda et al., 2019), whose specification is known to affect inverted fluxes materially (Chevallier, 2007). The two localization experiments (LOC_half and LOC_x2) change the localization length, which tapers the influence of an observation with distance through a Gaspari–Cohn function (operational half-width 0.025 rad, about 159 km, with the influence reaching zero near 320 km): LOC_half halves it to 0.0125 rad and LOC_x2 doubles it to 0.05 rad. A combined experiment (COMBINED) applies the transport, boundary, and observation-error perturbations together.

Recovery is summarized over the 60 matched cycles. Because the system cycles, the absolute recovery toward the target grows as corrections accumulate, so any single recovered fraction depends on which cycles are averaged. For each cycle, the posterior domain emission of a species, summed over domain, is expressed as a percentage of the prior inventory, P = 100 × Σ Epost / Σ Eprior (the uncorrected prior at 100 %, the target at 70 %); values closer to 70 % indicate stronger recovery. Table S2 reports this recovery for each experiment, including the known-truth baseline (CTL), as the mean and full range (minimum and maximum) across the cycles, and Fig. S15 shows its distribution. Because this absolute level depends on the averaging window, the sensitivity to each error source is shown separately as the paired per-cycle difference from the baseline, Δ = P(CTL) − P (Fig. S16, positive when the experiment recovers more than the baseline, in percentage points of the prior), which cancels the shared convergence trend and is therefore window-independent.

S1.2 Recovered-emission sensitivity to each error source

In every experiment the posterior moves from the prior toward the target (Fig. S15). The recovery is strongest near the observing stations, reaching about 65 % of the imposed bias for CO2 and 60 % for CH4 within 25 km of a station, and falls to near zero beyond about 150 km (Fig. S17). The domain-mean recovery is correspondingly low, and this is set by observation coverage: the three-station network samples only a small fraction of the domain, so emissions are constrained within the station footprints but left near the prior across the unobserved remainder (Kim et al., 2021).

That CO2 recovers more strongly than CH4, most clearly near the stations (Fig. S17), and is accompanied by larger inflation (Section S1.4), follows from the spatial structure of the emissions rather than from any difference in observation quality. The CO2 emission field is far more spatially concentrated: half of the total CO2 emission originates from about 1 % of the source cells, and the most-emitting 5 % of cells hold 80 % of the CO2 emission against 52 % of the CH4 emission, reflecting the dominance of point sources such as power generation and industry in CO2 as against the more distributed agricultural and waste sources of CH4. These point sources also lie closer to the observing network: about a third of the CO2 emission falls within 100 km of a station, against a fifth of the CH4 emission, so the stations sense a stronger emission signal for CO2 and the analysis is correspondingly more sensitive to it; we infer this to be the reason for its higher recovery. The same concentration also makes the CO2 emission-concentration covariance spikier, so the finite 20-member ensemble represents it with more sampling noise; this accounts for the larger CO2 inflation and the greater spatial variability of the CO2 increments (Section S1.4).

The localization length has the largest domain-mean signature in the matrix (Fig. S16, Table S2), but this signature is in part a property of the spatially uniform target perturbation. Because the imposed bias has the same sign everywhere, a wider localization spreads each station's increment across more of the unobserved domain and raises the domain-mean correction (LOC_x2), whereas a tighter cutoff confines it (LOC_half); the near-station recovery profiles show this directly (Fig. S18). The monotonic response therefore reflects how widely a fixed update is spread and does not by itself favor a wider cutoff; the operational 0.025 rad is matched to the station footprint scale. The asymmetry is sharper for CO2 (Fig. S18a): halving the cutoff not only confines the recovery but over-concentrates it on the nearest cells, raising the near-station recovery above CTL while driving the mid-range correction slightly the wrong way, as a point-source enhancement is attributed to too few cells; this cutoff therefore acts as a practical floor for the point-source-dominated CO2 field, below which recovery degrades rather than merely contracts. Transport error, representativeness error, and their combination shift the domain-mean recovery by less than about 2 % of the prior, and the combined experiment is sub-additive relative to the three individual perturbations, indicating limited interaction. The boundary-condition bias is the one structural perturbation with a consistent, species-specific signature, suppressing CH4 recovery near the stations (Section S1.3). Across the matrix, every perturbation changes the recovered domain emission by at most a few percent of the prior (the structural errors by under about 2 %, the localization sweep by under about 3.5 %), within the relative uncertainty already carried by the real-data posterior estimates of Section 6.4 (about ±7 % for CO2 and ±10 % for CH4).

S1.3 Boundary-condition bias reproduces the real-data CH4 signature

The BC experiment provides a controlled test of the interpretation offered for the real-data inversion, in which the CH4 vertical-profile discrepancy of Section 6.2 (Fig. 5) was interpreted as potentially reflecting residual boundary-condition bias, while not excluding possible missing-emission or transport-error contributions. The signature is clearest in the near-station recovery (Fig. S17): the imposed −30 ppb CH4 background bias makes BC the weakest-recovering CH4 experiment at every distance within the constrained region, as the biased inflow competes with the local emission signal where the network is informative. The effect is species-specific, the CO2 recovery being largely unaffected, as expected from a background bias proportionally smaller for CO2 (−2 ppm is about 0.5 % of the 414 ppm CO2 background) than for CH4 (−30 ppb is about 1.6 % of the 1920 ppb CH4 background). The domain-mean CH4 effect is small and within the cycle-to-cycle scatter (Table S2), consistent with a suppression localized to the footprints rather than spread across the domain. A systematic bias in the assimilated background propagates into the recovered fluxes (Masarie et al., 2011; Lauvaux et al., 2012), most strongly for column and vertical-profile diagnostics (Feng et al., 2019), to a degree set by the fraction of the domain the network constrains (Lauvaux et al., 2016). Because the imposed bias is of known size, the experiment provides a controlled scale for the sensitivity, at the level of a few percent of the prior under the tested configuration. It is a consistency check that supports the plausibility of the boundary-condition interpretation by showing that a boundary bias of the prescribed sign and magnitude can produce a CH₄-specific signature consistent with the real-data discrepancy. It is not presented as a unique attribution, nor as excluding possible missing-emission or transport-error contributions. The size of this effect depends on the prescribed CAMS-EGG4 inflow and can vary with the choice of lateral boundary product. Constraining the residual boundary-condition bias, for example with independent upwind or vertical-profile observations, is therefore an important consideration for future regional inversions.

S1.4 The filter is stable and the recovery is coverage-limited

The filter configuration is retained throughout and diagnosed rather than re-tuned. In every experiment the recovered emissions converge from the prior toward the target (Fig. S15, S17), while the ensemble variance contracts little per cycle, so the mean recovers through accumulated same-sign increments rather than spread collapse. The adaptive prior inflation (Anderson, 2009) stays modest and self-regulating without evidence of ensemble collapse (Fig. S19): at most about 1.19 for CO2 (operationally ≈ 1.11) and at most about 5 % above unity for CH4 (Fig. S19). This suggests that the tested OSSEs are not strongly under-dispersive. We therefore interpret the inflation behavior as a filter-stability diagnostic rather than as an optimization of the inflation scheme. The observation-error experiment (OE), which doubles the assigned observation error, does not degrade the domain-mean recovery (Table S2), suggesting that, under the tested observation-error perturbation, network coverage is the dominant limitation for domain-mean recovery.

These diagnostics characterize the present configuration used in the OSSE and real simulations directly. The complementary real-case diagnostics in Section S3 assess the practical sensitivity of the present system to ensemble size and inflation configuration, while a systematic assimilation-window comparison remains future work. The 20-member configuration shows no evidence of severe under-dispersion or spread collapse in the diagnostics used here. The localization sweep further diagnoses the sensitivity of recovered emissions to covariance tapering and spurious long-range correlations, which are known finite-ensemble sampling issues in EnKF applications (Kang et al., 2011, 2012; Anderson and Lei, 2013).

The prior-error structure spans two distinct forms here, the uniform scaling and the concentrated sector removal of the dense-network experiment; because spatially explicit fossil-fuel CO2 inventories, including the EDGAR inventory used here, differ appreciably in how they allocate emissions in space, both over the United States (Hutchins et al., 2017) and over East Asia, including China adjacent to the present domain (Yang et al., 2025), a finer sector-, region-, and time-resolved decomposition of the prior error, of the kind targeted by sector-resolved inversions (Miller and Michalak, 2017), is also left to future work.

S2 Controlled-twin evaluation of the inversion under a substantially denser observing-network scenario

Section S1.4 identified network coverage as the dominant limitation for domain-mean recovery under the tested observation-error and filter-stability diagnostics. With three stations the inversion constrains emissions within their footprints. This section tests how far the recovery improves once that constraint is relaxed, using a known-truth experiment that varies only the observing network, contrasting the operational three-station network with a substantially denser synthetic surface network under matched filter settings and an independent, strongly heterogeneous target (a single-sector emission removal). That target is CO2: its point-source dominance (Section S1.2, S1.4) is where a sparse network loses the most information, motivating a test of how much the system stands to gain from a substantially denser network.

Because the nature run and the assimilation again share the same physics, transport, and boundary conditions, the experiment isolates the information content of the denser network from the real-world errors examined in Section S1; it is an idealized diagnostic of how the emission constraint scales as the observing system grows, not a demonstration of grid-scale resolvability or operational accuracy. Adding surface stations is only one axis of that growth: diversifying the assimilated observation types (for example, satellite column retrievals and other platforms) is a complementary direction, left to future work as the system's forward operators are extended.

S2.1 Twin-experiment design imposing strong, heterogeneous target emission signals

The nature run (representing the “truth”) is generated with WRF-Chem using EDGAR CO2 emissions for March 2020. This truth simulation is performed without data assimilation and provides dynamically consistent meteorology and tracer fields used to generate synthetic observations. The OSSE is conducted for the period from 2020-03-03 to 2020-03-18, following a 48-hour spin-up (60 cycles of 6-hourly DA, starting at 00 UTC on 2020-03-03).

To define a clear and interpretable emission target for the OSSE, the corresponding inversion (prior) configuration uses EDGAR March 2020 CO2 emissions with the power-industry sector removed (“EDGAR without power-industry”). This idealized sector-removal setup provides a controlled test of how the implemented state-augmentation system responds to a known, spatially heterogeneous prior-emission perturbation (Fig. S20a). The removed EDGAR power-industry sector accounts for 36.5% of the domain-total CO2 emissions, increasing to 44.7% of emissions over South Korea, thereby imposing a substantial signal for the assimilation system to recover. Because the power-industry sector is concentrated at discrete point sources in industrial corridors rather than distributed uniformly, the experiment imposes a substantial, spatially heterogeneous signal that is, in principle, detectable. This ensures that the OSSE tests recovery of a clearly detectable prescribed prior-emission perturbation rather than detection of a marginal signal.

Two OSSEs are conducted under matched filter settings: (i) a 50-site network distributed primarily over South Korea and around major source regions (triangles in Fig. S20), with a minimum site spacing of approximately 30 km to increase sensitivity diversity while limiting redundancy; and (ii) a 3-site reference network corresponding to the current WMO/GAW station configuration (AMY/GSN/ULD) used in the real-data experiment. Synthetic hourly CO2 observations are sampled from the lowest model level of the nature run and assimilated within the same ±3 h temporal window used by the cycling system.

To emulate realistic measurement and representativeness uncertainty as in real applications, random observation error is introduced to each synthetic observation. These errors are assumed to be unbiased and Gaussian, specifically to match the real-data strategy described in Section 2.2: the error magnitude is dominated by the representativeness term and is parameterized as 5 % of the sampled mole fraction. Ensemble IC/BCs and emissions are randomly perturbed as described in Section 4 to introduce realistic background error relative to the unperturbed truth, while the deliberately imposed prior–truth difference is confined to the anthropogenic emission field. All other model inputs and ancillary datasets are kept identical to those in the real-data configuration to isolate the impact of observational density in a controlled twin setting.

S2.2 Assimilation configuration

This OSSE is intentionally configured as a flux-recovery diagnostic under perfect transport assumptions; meteorological variables are excluded from the augmented state to isolate the concentration–flux mapping and avoid conflating flux recoverability with transport uncertainty. The real-data experiments retain the full dual-state configuration. The assimilation step in the OSSE follows WRF-Chem/DART cycling framework described in Section 2: employing the EAKF with state augmentation to jointly estimate both GHG tracer field and surface flux parameter. The augmented state includes (i) 3-dimensional CO2 mixing ratios, and (ii) 2-dimensional anthropogenic flux scaling (or flux parameter) fields. The transport model configuration and meteorological IC/BCs follow the same setup as the nature run, with member-wise stochastic perturbations applied to the IC/BCs.

Compared to the baseline real-data configuration, a single adjustment is made for the OSSE to span the imposed truth–baseline emission gap. The only configuration parameter that differs is the prior emission uncertainty, specified as a 50 % multiplicative perturbation applied per grid cell to the EDGAR-without-power baseline. The configuration illustrates system behavior under enhanced observational coverage, using filter settings consistent with the real-data configuration (observation errors, localization, inflation) rather than idealized perfect-knowledge parameters. This design isolates the effect of expanded network density (the controlled variable) rather than conflating it with truth-tuned parameter optimization.

S2.3 Emission estimation under a dense surface network

OSSE performance is evaluated against the known truth using diagnostics that target emissions. We examine the South Korea–integrated CO2 emission budget to quantify recovery of the imposed missing-sector signal in the best-observed region. Under known-truth assumptions, the OSSE indicates that the assimilation partially recovers the national-scale CO2 emission budget. We report both the mean flux correction integrated over the OSSE period and the equivalent annualized rate. Over South Korea, the gap between the truth (“control”, ~688 Mt yr⁻¹) and prior emissions (~384 Mt yr⁻¹) is approximately 305 Mt yr⁻¹. Averaged over the OSSE cycling period, the posterior emission closes 264 Mt yr⁻¹ of this gap (a recovery rate of 86.5%). This is also observable in the cycle-to-cycle posterior trajectory in comparison with the 3-site reference OSSE run (with WMO/GAW sites; a recovery rate of 25.5%), posteriors rising further from baseline and the cycled prior toward the truth with the 50-site posterior reaching a markedly higher level. The dense network therefore increases the magnitude of the recovered correction in this idealized known-truth experiment (Fig. S21). Both experiments show similar cycle-scale variability patterns driven by shared synoptic-transport sensitivity rather than by the filter characteristics. This result provides an idealized reference for how national-scale emission recovery and uncertainty reduction may strengthen as surface observational coverage increases.

Spatial diagnostics from the OSSE indicate that, under known-truth assumptions, the assimilation partially recovers the spatial structure of the imposed emission signal (Fig. S20). The posterior emission increment (posterior–prior) exhibits a distribution that is broadly consistent with the “true difference” associated with the excluded power-industry emissions (compare Fig. S20a with Fig. S20d). The strongest adjustments occur in regions with the largest imposed emission differences and frequent downwind sampling by the expanded observing network. Increments remain negligible over remote cells far from the observation network (Fig. S20d), indicating that the prescribed localization effectively suppresses spurious distant corrections from sampling-noise correlations. Within the signal-bearing region, increments are predominantly positive, consistent with the imposed prior–truth direction, with no systematic wrong-direction corrections. Correspondingly, posterior emission uncertainty reductions, quantified by the decrease in ensemble spread (standard deviation) relative to the prior, are concentrated and strongest near the synthetic site clusters and major source regions, peaking above 60% over the densely observed source corridor over South Korea (Fig. S20c). The prior ensemble spread (Fig. S20b) is small but non-zero over the marginal sea cells, reflecting the imposed multiplicative emission perturbation; the analysis increments at these locations therefore lie within the prior uncertainty. Spatial maps of cycle-mean posterior emission error show that the assimilation directs its strongest emission updates toward the imposed point-source regions in both experiments (Fig. S22). In practice, localized DA transfers observational information into the emission field at the regional spatial scale set by the localization radius: with the imposed signal dominated by discrete large point sources, the innovations project as ensemble-supported regional corrections aligned with the underlying source structure rather than as cell-level adjustments at the individual point sources. Negative error magnitudes are systematically smaller in the 50-site experiment than in the 3-site reference, consistent with the higher national emissions recovery.

The controlled twin experiment provides a compact technical illustration of framework behavior under expanded observational coverage. Using a dense 50-site synthetic surface network, the system produces coherent analysis increments and spatially structured emission responses over the well-sampled region despite the large prescribed prior-truth emission gap, reflected in larger posterior uncertainty reduction. Because emission updates are formulated with a prior-proportional (multiplicative) error structure, recoverability is inherently limited where the perturbed emissions are low or near zero. The residual gap and broad posterior ensemble envelope reflect the difficulty of constraining the deliberately severe prior misspecification imposed here.

These results were achieved using filter settings consistent with the real-data configuration, rather than truth-tuned settings. Under known-truth assumptions, the augmented-state framework responds to increased observational information content in the expected direction. The OSSE therefore provides a controlled complement to the real-data demonstration, rather than a proof of operational robustness.

S3 Real-case sensitivity to ensemble size and inflation configuration

The controlled twin experiments in Sections S1 and S2 diagnose the response of the inversion to prescribed error sources and to observing-network density under known target conditions. To complement those idealized OSSE diagnostics, we provide two real-case sensitivity tests at the operational WMO/GAW sites: one that varies the ensemble size (January 2019) and one that varies the inflation configuration (January 2020). Both periods are performed for representative winter months, when strong anthropogenic signals and stable boundary-layer conditions make the ensemble-spread diagnostics most informative. These tests are not intended as an additional annual emission estimate; they are filter-configuration diagnostics that assess whether the ensemble-size and inflation settings used in the cycled WRF-Chem/DART system produce a self-consistent ensemble spread under real observational constraints.

The primary diagnostic is the ratio of the prior root-mean-square error (RMSE) to the total spread at the observation sites. The prior RMSE is the root-mean-square departure of the prior (forecast) ensemble mean from the observations, and the total spread combines the prior ensemble standard deviation in observation space with the prescribed observation-error standard deviation in quadrature (the square root of the sum of the two variances) (Desroziers et al., 2005; Fortin et al., 2014; Tandeo et al., 2020). A ratio near 1 indicates that the forecast spread and the observation error together are statistically consistent with the observed model-observation mismatch; a ratio above 1 indicates under-dispersion, or error unrepresented by the forecast ensemble, and a ratio below 1 indicates over-dispersion, an ensemble spread larger than the actual mismatch. The present online WRF-Chem/DART system uses 6 h cycling, with each analysis assimilating observations within a ±3 h window, so that each continuous in situ observation is used once and is not reused in adjacent analyses. Widening the observation window while retaining frequent analyses would move the system toward a smoother-like configuration and would require explicit treatment of observation reuse, temporal error correlation, and information double-counting. A systematic window-length comparison is therefore left to future work, most naturally when additional observation types with different temporal sampling, such as satellite retrievals, are introduced.

S3.1 Ensemble-size sensitivity

A finite ensemble samples the flow-dependent forecast-error covariance, so the ensemble size sets the quality of both the prior spread and the emission-concentration cross-covariances that drive the augmented-state EAKF update; smaller ensembles give noisier estimates and are more exposed to sampling error (Anderson, 2012). The January 2019 real-case cycling experiment is therefore repeated with 10, 20, and 30 ensemble members, and the prior RMSE/total-spread ratio is compared at AMY, ULD, and GSN for CO2 and CH4 (Fig. S23).

The three experiments share broadly similar temporal variability, indicating that the major mismatch events are set primarily by meteorological evolution, boundary inflow, and source-region influence rather than by ensemble size. The 10-member experiment, however, produces larger excursions above unity during several peak events, most clearly when the model-observation mismatch is large, consistent with the noisier spread and covariance estimates expected from a small ensemble. The 20- and 30-member experiments behave more alike, and increasing the ensemble from 20 to 30 members does not substantially change the temporal evolution of the ratio. This diagnostic does not identify a uniquely optimal ensemble size, but it shows that the 20-member configuration does not exhibit severe under-dispersion or spread collapse under the present observing configuration, whereas the 10-member configuration is more susceptible to unstable spread during high-mismatch events. The ensemble size can be revisited in future applications as the observing system expands and the computational power.

S3.2 Inflation sensitivity

Inflation broadens the ensemble variance during cycling to compensate for the insufficient spread that arises from finite ensemble size, sampling error, model error, and other unrepresented uncertainties (Anderson, 2009; El Gharamti, 2018). It acts by scaling the ensemble perturbations about their mean, leaving the ensemble mean unchanged. We test three configurations for January 2020: prior-only adaptive Gaussian state-space inflation (inf_flavor = 2), prior and posterior adaptive Gaussian state-space inflation (inf_flavor = 2), and prior-only enhanced adaptive inflation based on an inverse-gamma distribution (inf_flavor = 5). The first corresponds to the setting used in the annual inversion. Prior inflation broadens the forecast ensemble before the observations are assimilated and targets forecast-step uncertainty in transport, emissions, boundary conditions, and model physics, whereas posterior inflation is applied after the analysis and targets sampling and regression error and excessive spread reduction during the update (El Gharamti et al., 2019). The inverse-gamma option is an alternative adaptive formulation that lets the inflation and its variance evolve more flexibly than the Gaussian scheme while maintaining the prior ensemble spread.

The prior RMSE/total-spread ratios are similar across the three configurations at the observation sites during January 2020 (Fig. S24). The mean CO2 ratios are 0.848 (prior-only, inf_flavor = 2), 0.847 (prior and posterior, inf_flavor = 2), and 0.859 (prior-only, inf_flavor = 5); the corresponding CH4 ratios are 1.005, 1.005, and 1.018. Changing the inflation configuration therefore produces only minor changes in concentration-space spread consistency for this real-case diagnostic. Transient spikes occur at similar times in all three experiments, indicating that these events reflect common transport, meteorological, boundary-condition, or local-emission mismatches rather than a specific inflation formulation. These observation-space diagnostics are not interpreted as identifying an optimal inflation factor. In an augmented-state inversion, posterior emission increments also depend on the spread of the emission-state variables and on the ensemble-estimated concentration–emission cross-covariances. Thus, similar RMSE/total-spread ratios indicate comparable concentration-space consistency among the tested configurations, but do not imply that posterior emissions are insensitive to inflation settings.

Table S2. Recovery of the domain emission for each experiment, including the known-truth baseline (CTL): the mean and full range (minimum to maximum, in brackets) of the posterior domain emission over all 60 matched assimilation cycles, as a percentage of the prior inventory (the uncorrected prior at 100 %, the target at 70 %; values closer to 70 % indicate stronger recovery). The bracketed range is the cycle-to-cycle spread, large at the domain scale and shared across experiments, so the experiments recover to similar levels, with synoptic variability occasionally carrying individual cycles outside the 70–100 % band. The small per-experiment effects are isolated by the window-independent paired difference from CTL (Fig. S16), which removes this shared spread. The localization experiments bracket the recovery (LOC_half least, LOC_x2 most), while the structural errors (TR, BC, OE, and their combination) leave it within a few percent of the baseline.

| Experiment | Departure from CTL | CO2 (% of prior) | CH4 (% of prior) |
| --- | --- | --- | --- |
| CTL | Perfect-model baseline | 95.1 [77.7, 106.8] | 94.8 [74.9, 102.1] |
| TR | PBL scheme (Shin–Hong → MYNN2.5) | 94.5 [77.1, 106.1] | 94.0 [73.7, 105.3] |
| BC | Background bias (−2 ppm CO2, −30 ppb CH4) | 93.2 [73.7, 105.4] | 95.3 [77.6, 103.0] |
| OE | Observation error inflated ×2 (CO2 5→10 ppm, CH4 30→60 ppb) | 91.9 [80.3, 104.8] | 94.2 [80.9, 101.9] |
| COMBINED | TR + BC + OE combined | 92.1 [75.1, 106.0] | 94.8 [80.6, 105.0] |
| LOC_half | Localization radius halved (0.0125 rad, ≈80 km) | 98.5 [83.9, 111.6] | 98.4 [92.7, 101.3] |
| LOC_x2 | Localization radius doubled (0.05 rad, ≈319 km) | 91.1 [67.5, 106.7] | 92.0 [74.9, 106.0] |

Figure S15. Distribution of the posterior domain emission by error source over all 60 matched assimilation cycles, expressed as a percentage of the prior inventory (the uncorrected prior corresponds to 100 % and the target to 70 %; values closer to 70 % indicate stronger recovery), for (a) CO2 and (b) CH4. Every experiment lies below the prior, indicating recovery, and the localization experiments bracket the spread.

Figure S16. Sensitivity of the recovered emissions to each error source, shown as the paired per-cycle difference from the known-truth baseline (the recovery of each experiment relative to the baseline, Δ = P(CTL) − P), in percentage points of the prior, over all 60 matched assimilation cycles, for (a) CO2 and (b) CH4. Boxes give the median and interquartile range across cycles, with positive values indicating greater recovery than CTL; pairing against CTL removes the shared convergence trend, so that the comparison is independent of the averaging window.

Figure S17. Emission recovery as a function of distance to the nearest observation station, for (a) CO2 and (b) CH4, averaged over the matched cycles for the known-truth baseline and the structural-error experiments. Recovery reaches about 65 % (CO2) and 60 % (CH4) within 25 km of a station and declines to near zero beyond about 150 km, quantifying the spatial reach of the three-station network; the boundary-condition experiment (BC) shows the lowest CH4 recovery.

Figure S18. Emission recovery as a function of distance to the nearest station for the operational localization (CTL, 0.025 rad) and the halved (LOC_half, 0.0125 rad) and doubled (LOC_x2, 0.05 rad) cutoffs, for (a) CO2 and (b) CH4. A wider cutoff extends recovery farther from the network and a tighter cutoff confines it to the near field, the spatial counterpart of the domain-mean localization sensitivity in Table S2 and Fig. S16. For CO2 (a) the halved cutoff additionally drives mid-range corrections the wrong way, reflecting over-concentration of the update on the nearest cells.

Figure S19. Adaptive prior-inflation factor through the assimilation period for each of the seven experiments, for (a) CO2 and (b) CH4. The inflation stays modest and self-regulating in every experiment, reaching at most about 1.19 for CO2 and remaining at most about 5 % above unity for CH4; the ratio of posterior to prior ensemble spread (not shown) stays above about 0.9 throughout with no evidence of ensemble collapse, suggesting that the tested OSSEs are not strongly under-dispersive.

Figure S20. Temporal-mean OSSE diagnostics for CO2 emissions using the expanded 50-site synthetic surface network over the OSSE DA cycling period from 2020-03-03 to 2020-03-18. (a) Imposed target (“true difference” in CO2 emissions), defined as EDGAR (full) minus EDGAR without the power-industry sector. (b) Prior emission uncertainty, expressed as the time-averaged prior ensemble spread (standard deviation) of CO2 emissions. (c) Posterior uncertainty reduction for CO2 emissions expressed as the percent decrease in ensemble spread (standard deviation) relative to the prior. (d) Analysis increment (posterior minus prior) in CO2 emissions from the OSSE inversion. Triangles denote the locations of the 50 synthetic surface observation sites. All emission panels are shown in flux unit of MtCO2 km⁻² yr⁻¹. Emissions are displayed on a logarithmic color scale to visualize the highly skewed, point-source-dominated imposed signal, and panel (d) is displayed on a symmetric diverging logarithmic color scale; red (blue) shading indicates an increase (decrease) in the posterior relative to the prior.

Figure S21. Domain-total CO2 emission convergence over South Korea during the 15 days of OSSE cycling window (03-18 March 2020) for (a) the 50-site experiment and (b) the 3-site (AMY/GSN/ULD) reference experiment. Both experiments use identical filter settings (N = 20 ensemble members, 0.025 rad / ~159 km localization half-width, 50 % prior emission uncertainty); the only difference between panels is the number of synthetic observation sites. Curves show: the truth (black dashed, daily mean of the full EDGAR reference field), the initial baseline (gray dotted, EDGAR with the power-industry sector removed, defining the imposed truth–prior emission gap to be recovered), the cycled prior ensemble mean (thin solid), and the posterior ensemble mean (thick solid with circle markers); the shaded band shows the posterior ±1σ ensemble spread. Time-averaged emission-space recovery rate over the cycling window, defined as (posterior – baseline) / (truth – baseline): 86.5 % for the 50-site experiment and 25.5 % for the 3-site reference experiment.

Figure S22. Cycle-mean posterior CO2 emission error (posterior − truth, MtCO2 km⁻² yr⁻¹) averaged over the OSSE cycles (3–18 March 2020), for (a) the 50-site experiment and (b) the 3-site (AMY/GSN/ULD) reference experiment. Color scale is symmetric logarithmic; red (blue) shading indicates posterior emissions higher (lower) than truth.

Figure S23. Time series of the prior RMSE/total-spread ratio for CO2 (a-c) and CH4 (d-f) at AMY (a, d), ULD (b, e), and GSN (c, f) during January 2019 for the 10-member (blue solid line), 20-member (orange dashed line), and 30-member (purple dash-dotted line) ensemble experiments. The black dashed line marks a ratio of 1, and the colored dashed lines show the mean ratio for each ensemble size.

Figure S24. Time series of the prior RMSE/total-spread ratio for CO2 (a) and CH4 (b) at the observation sites during January 2020 for the three inflation configurations of Section S3.2: prior only (inf_flavor=2; blue solid line), prior and posterior (inf_flavor=2; orange dashed line), prior only (inf_flavor=5, purple dash-dotted line).

References

Agustí-Panareda, A., Diamantakis, M., Massart, S., Chevallier, F., Muñoz-Sabater, J., Barré, J., Curcoll, R., Engelen, R., Langerock, B., Law, R. M., Loh, Z., Morguí, J. A., Parrington, M., Peuch, V.-H., Ramonet, M., Roehl, C., Vermeulen, A. T., Warneke, T., and Wunch, D.: Modelling CO2 weather – why horizontal resolution matters, Atmos. Chem. Phys., 19, 7347–7376, https://doi.org/10.5194/acp-19-7347-2019, 2019.

Anderson, J. L.: Spatially and temporally varying adaptive covariance inflation for ensemble filters, Tellus A, 61, 72–83, https://doi.org/10.1111/j.1600-0870.2008.00361.x, 2009.

Anderson, J. L.: Localization and Sampling Error Correction in Ensemble Kalman Filter Data Assimilation, Monthly Weather Review, 140, 2359–2371, https://doi.org/10.1175/MWR-D-11-00013.1, 2012.

Anderson, J. L. and Lei, L.: Empirical localization of observation impact in ensemble Kalman filters, Mon. Weather Rev., 141, 4140–4153, https://doi.org/10.1175/MWR-D-12-00330.1, 2013.

Bisht, J. S. H., Patra, P. K., Takigawa, M., Sekiya, T., Kanaya, Y., Saitoh, N., and Miyazaki, K.: Estimation of CH4 emission based on an advanced 4D-LETKF assimilation system, Geosci. Model Dev., 16, 1823–1838, https://doi.org/10.5194/gmd-16-1823-2023, 2023.

Callewaert, S., Zhou, M., Langerock, B., Wang, P., Wang, T., Mahieu, E., and De Mazière, M.: A WRF-Chem study of the greenhouse gas column and in situ surface mole fractions observed at Xianghe, China – Part 2: Sensitivity of carbon dioxide (CO2) simulations to critical model parameters, Atmos. Chem. Phys., 26, 899–921, https://doi.org/10.5194/acp-26-899-2026, 2026.

Chen, H. W., Zhang, F., Lauvaux, T., Scholze, M., Davis, K. J., and Alley, R. B.: Regional CO2 inversion through ensemble-based simultaneous state and parameter estimation: TRACE framework and controlled experiments, J. Adv. Model. Earth Syst., 15, e2022MS003208, https://doi.org/10.1029/2022MS003208, 2023.

Chevallier, F.: Impact of correlated observation errors on inverted CO2 surface fluxes, Geophys. Res. Lett., 34, https://doi.org/10.1029/2007GL030463, 2007.

Custódio, D., Borrego, C., and Relvas, H.: Worldwide evaluation of CAMS-EGG4 CO2 data re-analysis at the surface level, Toxics, 10, 331, https://doi.org/10.3390/toxics10060331, 2022.

Deng, A., Lauvaux, T., Davis, K. J., Gaudet, B. J., Miles, N., Richardson, S. J., Wu, K., Sarmiento, D. P., Hardesty, R. M., Bonin, T. A., Brewer, W. A., and Gurney, K. R.: Toward reduced transport errors in a high resolution urban CO2 inversion system, Elem. Sci. Anth., 5, 20, https://doi.org/10.1525/elementa.133, 2017.

Desroziers, G., Berre, L., Chapnik, B., and Poli, P.: Diagnosis of observation, background and analysis-error statistics in observation space, Q. J. R. Meteorol. Soc., 131, 3385–3396, https://doi.org/10.1256/qj.05.108, 2005.

Díaz-Isaac, L. I., Lauvaux, T., and Davis, K. J.: Impact of physical parameterizations and initial conditions on simulated atmospheric transport and CO2 mole fractions in the US Midwest, Atmos. Chem. Phys., 18, 14813–14835, https://doi.org/10.5194/acp-18-14813-2018, 2018.

El Gharamti, M.: Enhanced adaptive inflation algorithm for ensemble filters, Mon. Weather Rev., 146, 623–640, https://doi.org/10.1175/MWR-D-17-0187.1, 2018.

El Gharamti, M., Raeder, K., Anderson, J., and Wang, X.: Comparing adaptive prior and posterior inflation for ensemble filters using an atmospheric general circulation model, Mon. Weather Rev., 147, 2535–2553, https://doi.org/10.1175/MWR-D-18-0389.1, 2019.

Feng, S., Lauvaux, T., Davis, K. J., Keller, K., Zhou, Y., Williams, C., Schuh, A. E., Liu, J., and Baker, I.: Seasonal characteristics of model uncertainties from biogenic fluxes, transport, and large-scale boundary inflow in atmospheric CO2 simulations over North America, J. Geophys. Res.-Atmos., 124, 14325–14346, https://doi.org/10.1029/2019JD031165, 2019.

Fortin, V., Abaza, M., Anctil, F., and Turcotte, R.: Why should ensemble spread match the RMSE of the ensemble mean?, J. Hydrometeorol., 15, 1708–1713, https://doi.org/10.1175/JHM-D-14-0008.1, 2014.

Hoffman, R. N. and Atlas, R.: Future observing system simulation experiments, Bull. Am. Meteorol. Soc., 97, 1601–1616, https://doi.org/10.1175/BAMS-D-15-00200.1, 2016.

Hutchins, M. G., Colby, J. D., Marland, G., and Marland, E.: A comparison of five high-resolution spatially-explicit, fossil-fuel, carbon dioxide emission inventories for the United States, Mitig. Adapt. Strateg. Glob. Change, 22, 947–972, https://doi.org/10.1007/s11027-016-9709-9, 2017.

Kang, J.-S., Kalnay, E., Liu, J., Fung, I., Miyoshi, T., and Ide, K.: “Variable localization” in an ensemble Kalman filter: Application to the carbon cycle data assimilation, J. Geophys. Res.-Atmos., 116, D09110, https://doi.org/10.1029/2010JD014673, 2011.

Kang, J.-S., Kalnay, E., Miyoshi, T., Liu, J., and Fung, I.: Estimation of surface carbon fluxes with an advanced data assimilation methodology, J. Geophys. Res. Atmos., 117, D24101, doi:10.1029/2012JD018259, 2012.

Kim, J., Polavarapu, S. M., Jones, D. B. A., Chan, D., and Neish, M.: The resolvable scales of regional-scale CO2 transport in the context of imperfect meteorology: the predictability of CO2 in a limited-area model, J. Geophys. Res.-Atmos., 126, e2021JD034896, https://doi.org/10.1029/2021JD034896, 2021.

Lauvaux, T., Schuh, A. E., Uliasz, M., Richardson, S., Miles, N., Andrews, A. E., Sweeney, C., Diaz, L. I., Martins, D., Shepson, P. B., and Davis, K. J.: Constraining the CO2 budget of the corn belt: exploring uncertainties from the assumptions in a mesoscale inverse system, Atmos. Chem. Phys., 12, 337–354, https://doi.org/10.5194/acp-12-337-2012, 2012.

Lauvaux, T., Miles, N. L., Deng, A., Richardson, S. J., Cambaliza, M. O., Davis, K. J., Gaudet, B., Gurney, K. R., Huang, J., O’Keefe, D., Song, Y., Karion, A., Oda, T., Patarasuk, R., Razlivanov, I., Sarmiento, D., Shepson, P., Sweeney, C., Turnbull, J., and Wu, K.: High-resolution atmospheric inversion of urban CO2 emissions during the dormant season of the Indianapolis Flux Experiment (INFLUX), J. Geophys. Res.-Atmos., 121, 5213–5236, https://doi.org/10.1002/2015JD024473, 2016.

Liang, R., Zhang, Y., Chen, W., Zhang, P., Liu, J., Chen, C., Mao, H., Shen, G., Qu, Z., Chen, Z., Zhou, M., Wang, P., Parker, R. J., Boesch, H., Lorente, A., Maasakkers, J. D., and Aben, I.: East Asian methane emissions inferred from high-resolution inversions of GOSAT and TROPOMI observations: a comparative and evaluative analysis, Atmos. Chem. Phys., 23, 8039–8057, https://doi.org/10.5194/acp-23-8039-2023, 2023.

Liu, Y., Kalnay, E., Zeng, N., Asrar, G., Chen, Z., and Jia, B.: Estimating surface carbon fluxes based on a local ensemble transform Kalman filter with a short assimilation window and a long observation window, Geosci. Model Dev., 12, 2899–2914, https://doi.org/10.5194/gmd-12-2899-2019, 2019.

Locatelli, R., Bousquet, P., Chevallier, F., Fortems-Cheney, A., Szopa, S., Saunois, M., Agusti-Panareda, A., Bergmann, D., Bian, H., Cameron-Smith, P., Chipperfield, M. P., Gloor, E., Houweling, S., Kawa, S. R., Krol, M., Patra, P. K., Prinn, R. G., Rigby, M., Saito, R., and Wilson, C.: Impact of transport model errors on the global and regional methane emission estimates, Atmos. Chem. Phys., 13, 9917–9937, https://doi.org/10.5194/acp-13-9917-2013, 2013.

Masarie, K. A., Pétron, G., Andrews, A., Bruhwiler, L., Conway, T. J., Jacobson, A. R., Miller, J. B., Tans, P. P., Worthy, D. E., and Peters, W.: Impact of CO2 measurement bias on CarbonTracker surface flux estimates, J. Geophys. Res.-Atmos., 116, D17305, https://doi.org/10.1029/2011JD016270, 2011.

Michalak, A. M., Randazzo, N. A., and Chevallier, F.: Diagnostic methods for atmospheric inversions of long-lived greenhouse gases, Atmos. Chem. Phys., 17, 7405–7421, https://doi.org/10.5194/acp-17-7405-2017, 2017.

Miller, S. M., Hayek, M. N., Andrews, A. E., Fung, I., and Liu, J.: Biases in atmospheric CO2 estimates from correlated meteorology modeling errors, Atmos. Chem. Phys., 15, 2903–2914, https://doi.org/10.5194/acp-15-2903-2015, 2015.

Miller, S. M. and Michalak, A. M.: Constraining sector-specific CO2 and CH4 emissions in the US, Atmos. Chem. Phys., 17, 3963–3985, https://doi.org/10.5194/acp-17-3963-2017, 2017.

Park, C., Park, S., Gurney, K. R., Gerbig, C., DiGangi, J. P., Choi, Y., and Lee, H. W.: Numerical simulation of atmospheric CO2 concentration and flux over the Korean Peninsula using WRF-VPRM model during KORUS-AQ 2016 campaign, PLoS ONE, 15, e0228106, https://doi.org/10.1371/journal.pone.0228106, 2020.

Segato, D., Graziosi, F., and Manca, G.: Evaluation of Copernicus Atmosphere Monitoring Service methane products 2019–2023, Publications Office of the European Union, Luxembourg, https://doi.org/10.2760/8653047, 2025.

Tandeo, P., Ailliot, P., Bocquet, M., Carrassi, A., Miyoshi, T., Pulido, M., and Zhen, Y.: A review of innovation-based methods to jointly estimate model and observation error covariance matrices in ensemble data assimilation, Mon. Weather Rev., 148, 3973–3994, https://doi.org/10.1175/MWR-D-19-0240.1, 2020.

Tang, W., Arellano, A. F., DiGangi, J. P., Choi, Y., Diskin, G. S., Agustí-Panareda, A., Parrington, M., Massart, S., Gaubert, B., Lee, Y., Kim, D., Jung, J., Hong, J., Hong, J.-W., Kanaya, Y., Lee, M., Stauffer, R. M., Thompson, A. M., Flynn, J. H., and Woo, J.-H.: Evaluating high-resolution forecasts of atmospheric CO and CO2 from a global prediction system during KORUS-AQ field campaign, Atmos. Chem. Phys., 18, 11007–11030, https://doi.org/10.5194/acp-18-11007-2018, 2018.

van der Velde, I. R., Miller, J. B., van der Molen, M. K., Tans, P. P., Vaughn, B. H., White, J. W. C., Schaefer, K., and Peters, W.: The CarbonTracker Data Assimilation System for CO2 and δ13C (CTDAS-C13 v1.0): retrieving information on land–atmosphere exchange processes, Geosci. Model Dev., 11, 283–304, https://doi.org/10.5194/gmd-11-283-2018, 2018.

Vardag, S. N. and Maiwald, R.: Optimising urban measurement networks for CO2 flux estimation: a high-resolution observing system simulation experiment using GRAMM/GRAL, Geosci. Model Dev., 17, 1885–1902, https://doi.org/10.5194/gmd-17-1885-2024, 2024.

Yang, H., Wu, K., Shen, H., Janssens-Maenhout, G., Crippa, M., Guizzardi, D., and Zhou, M.: A comparative analysis of China’s anthropogenic CO2 emissions (2000–2023): insights from six bottom-up inventories and uncertainty assessment, Atmos. Chem. Phys., 25, 18111–18127, https://doi.org/10.5194/acp-25-18111-2025, 2025.