New Table layouts:

Table 4: Transport data and sources for (a) modes and (b) route. Variations used from actual data, include: Buses: 85% diesel of fleet (in 2015) and rest mostly hybrid; Electric (EV) and low-emission vehicles: EV cars 0.2% of GL registered vehicles (2015) (DfT and DVLA, 2019). Data sources: §8 - ONS (2014b); §9 - ONS (2018); §10 - DfT (2017); §11 - DfT (2014a, 2014b); §12 - London Datastore (2014); §13 - OS (2016); §14 - Smith et al. (2009); §15 - Highways Agency (2017); §16 - TfL (2018); §17 - TfL (2019); §18 - OS (2015); §19 - TfL Train and Underground Rolling Stock Information Sheets from §10; §20 - TfL working timetables from §10; §21 - Iamarino et al. (2012). ΔNot applied in evaluation

| (a) Mode m | People vehicle-1 §10, §15 | QF,M person-1 (W m-2) §21 | Fuel Use Ratio §10, §17 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Petrol | Diesel |
| Car | 1.4 | 70 | 0.84 | 0.16 |
| Van | 1.4 | 70 | 0.1 | 0.9 |
| Taxi | 2.5 | 70 | 0 | 1 |
| Motorcycle | 1 | 70 | 1 | 0 |
| Bus | 17.3 | 55 | 0 | 1 |
| UG rail | -- | 62Δ | -- | -- |
| Surface rail | -- | 55 Δ | -- | -- |
| Bicycle | 1 | 230 | -- | -- |
| Walking | 1 | 140 | -- | -- |

| (b) Route (data vary) | Source |
| --- | --- |
| AADT (road vehicles) | §12 |
| vr,lim (road vehicles) | §18 |
| Fm,f by speed & fuel for road vehicles (electric not considered in this evaluation) | §9, §14 |
| Commute mode choice (aDi→aWj) | §8 |
| Journey Time | §10 |
| Cm,r, Rlim | §11, §19 Δ, §20 Δ |
| Route (r) dimensions (e.g. length, no. of lanes, no. of tracks) | §12, §13, §16, §19 Δ |

Table C2: Properties used in STEBBS vary by property age (<1965, > 1965), type (HB - house & bungalow; flat), component (roof etc) for (a) building fabric and external ground, and (b) DHW.  L: thickness (m); ε: emissivity; τ: effective transmissivity; Θ: surface albedo; ke: effective thermal conductivity (W m-1 K-1); ⍴: density (kg m-3); cp: specific heat of air at constant pressure (J kg-1 K-1) (Internal Air 1005 ); h: convection coefficient (Int: internal, Ext: external) (W m-2 K-1); VFR: volumetric flow rate of DHW per water user (dom: domestic, n-dom: non-domestic) (10-3 m3 s-1); VR: ventilation rate (10-3 m3 s-1); VT: DHW tank volume (m3); WWR: window-to-wall ratio (0.4). Vessels: all other storage of DHW. For data sources refer to Table 3. a varies with wind speed.

| - Building fabric |  | Roof & wall |  | Window | Ground Floor | Internal Mass | External Ground |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HB | Flat |  |  |  |  |
| L | < 1965 | 0.241 | 0.327 | 0.005 | 0.5 | - | 2 |
|  | > 1965 | 0.373 | 0.373 | 0.02 | 0.5 | - |  |
| ke | < 1965 | 0.837 | 0.835 | 1.05 | 0.752 | 0.121 | 1.28 |
|  | > 1965 | 0.104 | 0.104 | 0.041 | 0.690 | 0.121 |  |
| r | < 1965 | 1692 | 1690 | 2500 | 1540 | 873.7 |
|  | > 1965 | 1076 | 1076 | 1000.7 | 1470 | 873.7 |
| cp | < 1965 | 803.1 | 804.1 | 840 | 1012.8 | 967.9 |
|  | > 1965 | 865.9 | 865.9 | 902.4 | 1016 | 967.9 |
| h | Int. | 3 | 3 | 3 | 2.8 | 3 |
|  | Ext. | vara | vara | vara | - | - |
| Θ |  | 0.6 | 0.6 | 0.05 | - | 0 |
| e |  | 0.9 | 0.9 | 0.88 | - | 0.91 |
| τ |  | 0 | 0 | 0.9 | - | 0 |
| VR |  | 600 |  |  |  |  |

| (b) DHW |  | Tank |  |  | Vessel |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L (m) |  | 0.055 |  |  | 0.0047 |  |  |
| e |  | 0.9 |  |  | 0.91 |  |  |
| ke |  | 0.0275 |  |  | 0.16 |  |  |
| r |  | 745.55 |  |  | 1380 |  |  |
| cp |  | 1380 |  |  | 1380 |  |  |
| h | Int. | 243 |  |  | 243 |  |  |
|  | Ext. | 3 |  |  | 3 |  |  |
| VFR | dom | 0.183 |  |  | 0.1372 |  |  |
|  | n-dom | 0.15 |  |  | 0.1125 |  |  |
| People§ residence-1 |  | 1 | 2 | 3 | 4 | 5 | 6 |
| VT (m3) |  | 0.115 | 0.115 | 0.125 | 0.148 | 0.17 | 0.18 |